The International Journal of

Pavement Engineering & Asphalt Technology

Volume 23                             ISSN 1464-8164                                  March 2022

For:

- Roads,Highways,  City Streets & Airports

- Production and Use of: Concrete & Asphalt Pavements and Bituminous Materials in Civil Engineering and Building Construction

- Design,   Manufacturing, Specification, Management, Construction, Evaluation and  Rehabilitation

- IT

in Pavement Engineering, Material Technology & Research Focus

Editor-in-Chief:  Professor Hassan Al Nageim

Editors:

Dr Michael Nunn

Professor Shakir Al-Busaltan

Assist. Prof Anmar Dulaimi

K. Karyono

20th Annual International Conference on Highways and Airport Pavement Engineering, Asphalt Technology, and Infrastructure International Conference

March 2022, Adaptive Learning Zone Liverpool John Moores University, Liverpool, UK

|  | The International Journal of Pavement Engineering & Asphalt Technology, PEAT ISSN 1464-8164 |  |
| --- | --- | --- |
| - Roads, City Streets, Highways & Airports - Production and Use of: Concrete Pavement and Bituminous Materials in Civil Engineering and Building Construction - Design, Manufacturing, Specifications, Management, Construction, Evaluations, Rehabilitation, - IT in Pavement Engineering, Material Technology & Research Focus |  |  |

Call for Papers

The International Journal of Pavement Engineering & Asphalt Technology has been established as a leading refereed forum. It reports on innovative research; design breakthroughs; construction practices; evaluation techniques; industrial applications; specification & analysis of asphalt binders & mixtures.  Authors are invited to submit papers for inclusion in future editions. Theoretical papers should deal with innovation and, wherever possible, the implementation of new materials or techniques. Papers describing work in progress should discuss the long-term practical implications of the research undertaken. Contributions are especially welcome from designers and contractors dealing with new materials or construction / testing techniques (including destructive and non-destructive testing). Papers discussing IT, construction management, environmental maintenance and pavement material performance will also be considered.

March 2022

The International Journal of Pavement Engineering & Asphalt Technology (ISSN: 1464-8164) is published annually. All rights including translation reserved. The Editors and members of the Journal’s Editorial Review Board are from a range of organisations including academic institutions, oil and chemical producers, government institutions, consulting companies and equipment developers with a wide range of expertise.

Papers or other contributions and the statements made, or opinions expressed therein are published on the understanding that the author of the contribution is solely responsible for the opinion expressed in it and its publication does not imply that such statements and / or opinions are, or reflect the views or opinions of, the Editors, the Editorial Review Board or the Publisher. The views expressed in the Journal do not represent any of the organisations for which the Editors or members of the Editorial Review Board are associated.

No part of this publication may be reproduced, stored in a retrieval system, or transmitted in any form or by any means electrical, mechanical, photocopy, recording or otherwise, without the prior written permission of the Editor –in- Chief.

|  | The International Journal of Pavement Engineering & Asphalt Technology, PEAT ISSN 1464-8164 |  |
| --- | --- | --- |
| #### For - Roads, City Streets & Airports - Production and Use of: Concrete Pavement and Bituminous Materials in Civil Engineering And Building Construction - Design, Manufacturing, Specification, Management, Construction, Evaluation, Rehabilitation, - IT in Pavement Engineering, Material Technology & Research Focus |  |  |

| TABLE OF CONTENTS |  |
| --- | --- |
| Editors | 5 |
| Editorail Board | 6-7 |
| Editorail | 8 |
| TECHNICAL PAPERS | 9 |
| A STUDY ON USE OF WASTE PLASTICS IN SMA MIXES: EFFECT OF PLASTIC TYPE, SIZE, AND DOSAGE Rajan Choudhary1*, Pankaj Kumar Singh2, Ankush Kumar3 and Santanu Pathak3 1Professor, Department of Civil Engineering, Indian Institute of Technology Guwahati (IIT Guwahati), India * Corresponding Author, Email: rajandce@iitg.ac.in 2Former Post-graduate Student, Department of Civil Engineering, IIT Guwahati, India 3PhD Research Scholar, Department of Civil Engineering, IIT Guwahati, India | 10-22 |
| CLASSIFICATION OF UNDERGROUND CAVITY BASED ON GEOMETRIC PARAMETERS FROM GROUND PENETRATING RADAR DATA Carlo Elipse1: PhD Candidate, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 6499 4501, Email: carloelipse@gmail.com Julius Marvin Flores2: Research and Development Department Manager, IRIS Technology, Seoul, South Korea, Tel: +82 10 2168 1434, Email: juliusmarvinflores@gmail.com Van Phuc Tran2: PMS Department Team Leader, IRIS Technology, Seoul, South Korea, Tel: +82 10 4964 2168, Email: vanphuctran053@gmail.com Son Dong Nguyen1: PhD Candidate, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 8461 3504, Email: nguyensondong88@gmail.com Hyun Jong Lee1*: Professor, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 5360 2419, Email: hlee@sejong.ac.kr | 23-37 |
| ASPHALT REINFORCEMENT – A PROVEN ECONOMIC & ECOLOGICAL ASHPALT REHABILTIATION METHOD Andreas Elsing Dipl.-Ing., HUESKER Synthetic GmbH, Fabrikstrasse 13-15, Gescher, 48712, Germany, Tel: +49 2542 7010, elsing@huesker.de Christoph Hessing Dipl.-Ing., HUESKER Synthetic GmbH, Fabrikstrasse 13-15, Gescher, 48712, Germany, Tel: +49 2542 7010, hessing@huesker.de Graham Horgan Director, Huesker Limited, 1 Quay Business Centre, Warrington, WA2 8LT, UK Tel: +49 1925 629393, grahamh@huesker.co.uk | 38-54 |
| The Use of Repetitive Strain Oscillation to Measure Relaxation Capacity in Bitumen F. Walmsley; R. Haigh; and G. Schofield Total Energies UK Limited, Bitumen Division, Preston, UK | 55-71 |
| Thermal Fatigue Analysis of Pavement Design using Hydrated Lime Modified Asphalt Concrete Yu Wang 1*, Azedin Al Ashaibi 1, Amjad Albayati 2, Jonathan Haynes 1, 1 School of Science, Engineering & Environment, University of Salford, UK 2 Department of Civil Engineering, University of Baghdad, Iraq *Corresponding author: y.wang@salford.ac.uk | 72-88 |
| Content of Previos Issues | 89-108 |
| Information for Prospective Authors | 109-110 |
| Subscription Form | 111 |
| Instruction to Make Online Payment for Subscription | 112 |

|  | The International Journal of Pavement Engineering & Asphalt Technology, PEAT ISSN 1464-8164 |  |
| --- | --- | --- |
| #### For - Roads, City Streets & Airports - Production and Use of: Concrete Pavement and Bituminous Materials in Civil Engineering And Building Construction - Design, Manufacturing, Specification, Management, Construction, Evaluation, Rehabilitation, - IT in Pavement Engineering, Material Technology & Research Focus |  |  |

EDITORS

Professor Hassan Al Nageim

Liverpool John Moores University, UK

Editor-in-Chief

Dr. Michael Nunn

Transport & Road Research Laboratory, UK

Editor

Assist. Prof Anmar Dulaimi

University of Warith Al-Anbiyaa, Iraq;

Liverpool John Moores University, UK, Editor

Professor Shakir Al-Busaltan

Department of Civil Engineering, University of Kerbala, Iraq, Editor

K. Karyono

Liverpool John Moores University, UK

Universitas Multimedia Nusantara, Indonesia, Editor

EDITORIAL BOARD

| Professor A. Woodside, Emeritus Professor in Highway Engineering, Ulster University, UK |  | Mr. I. Walsh, Head of Laboratories, Babtie Laboratory, UK |
| --- | --- | --- |
| Mr. F. Chaignon, Department of Research & Development, Colas, France |  | Dr. G. White, University of the Sunshine Coast, Australia |
| Mr. D. Day, Nynas Bitumen, UK |  | Dr. R. Douglas, University of Canterbury, New Zealand |
| Dr. Manar Hamzah Herez Department of Civil Engineering, Kufa University, Iraq |  | Dr. Hassnen M. Jafer Civil Engineering Department, College of Engineering, University of Babylon, Iraq |
| Dr. M.N.S. Hadi, Faculty of Engineering, University of Wollongong, Australia |  | Dr. L. Gaspar, KTI, Hungary |
| Dr. R. Hunter, Consulting Civil Engineers, Hunter & Edgar, UK |  | Professor E. Santagata, Politecnico di Torino, Torino, Italy |
| Dr. Hayder Kamil Shanbara Department of Civil Engineering, Al-Muthanna University, Iraq |  | Dr. Abbas Al-Hdabi Civil Engineering Department, Faculty of Engineering, Kufa University, Iraq |
| Dr. J.M. Reid, Senior Technical Manager, TRL, UK |  | Dr. I.L. Jamieson, Ireland |
| Dr. J.C. Nicholls, UK |  | Dr. R.J. Ellis, Aggregate Industries UK Limited |
| Assist. Prof. Basim Hassan Al-Humeidawi Roads and Transport Department, College of Engineering, University of Al-Qadisiyah, Iraq |  | Dr. Tariq Al-Mansoori Department of Civil Engineering, Al-Muthanna University, Iraq |
| Dr. S. Said, Swedish National Road & Transport Research Institute, Sweden |  | Professor A.F. Nikolaides, Department of Civil Engineering, University of Thessaloniki, Greece |
| Dr. W. Woodword, Highway Engineering Research Group, Ulster University, UK |  | Dr. R.A. Peden, Tennants Tar Distillers, UK |
| Assist. Prof. Dr. Jalal Al-Obaedi Roads and Transport Department, College of Engineering, University of Al-Qadisiyah, Iraq |  | Dr. Mahdi Al-Naddaf Dept. of Civil Engineering, College of Engineering, the University of Karbala, Iraq. |
| Professor M. Pasetto, University of Padua, Italy |  | Ms. E.S. Caballero, R&D Investigator Acciona Infrastructures, Spain |
| Dr. Hamid Athab Eedan Al-Jameel College of Engineering, University of Kufa, Iraq |  | Dr. Alaa M Shaban Department of Civil Engineering, Engineering College, University of Kerbala, Iraq |
| Dr. Anmar Dulaimi Department of Civil Engineering, University of Warith Alanbiyaa, Iraq; Department of Civil Engineering, Liverpool John Moores University, UK |  | Assist. Prof. Dr. Shakir Al-Busaltan Department of Civil Engineering, University of Kerbala, Iraq |

EDITORIAL

###### Liverpool Centre for Materials Technology

20th Annual International Conference on Highways and Airport Pavement Engineering, Asphalt Technology, and Infrastructure International Conference

April 2022, Adaptive Learning Zone Liverpool John Moores Univ., Liverpool, UK

SPONSORED & SUPPORTED BY:

ASI Solutions Colas Ltd Total Energies WDM ltd  Velocity UK Highways England The Chartered Institution of Highways and Transportation (CIHT) The Institution of Asphalt Technology (IAT)The Institution of Civil Engineers (ICE), UK

LJMU 2022 Conference

The Department of Civil Engineering at Liverpool John Moores University in association with ASI Solutions and Colas Ltd is hosting its 20th Annual International Conference addressing: Highways and Airport Pavement Engineering, Asphalt Technology, and Infrastructure International Conference.

Conference theme

Conference is aimed at stakeholders with specific interest in the; development, construction and management of asphalt technology, sustainable infrastructure, environmental protection and energy reduction, aggregate recycling initiatives, airport and highways design and maintenance. The conference will be of interest to; policy advisors, environmental regulators, infrastructure clients, specifiers, planners, designers, local authorities, highway related consultants and designers, materials suppliers, construction companies, contractors and educational institutions.

WHO SHOULD ATTEND?

The conference will be of interest  to; policy advisors, environmental  regulators, infrastructure clients, specifiers, planners, designers, local authorities, highway related consultants and designers, materials suppliers, construction companies, contractors and educational institutions.

PUBLICATIONS

The papers will be reviewed by the conference scientific and technical board and published in the conference proceedings. Selected papers will also be refereed and published in a special issue of the International Journal of Pavement Engineering and Asphalt Technology, ISBN 1464-184.

TECHNICAL PAPERS

A STUDY ON USE OF WASTE PLASTICS IN SMA MIXES: EFFECT OF PLASTIC TYPE, SIZE, AND DOSAGE

Rajan Choudhary1*, Pankaj Kumar Singh2, Ankush Kumar3 and Santanu Pathak3

1Professor, Department of Civil Engineering, Indian Institute of Technology Guwahati (IIT Guwahati), India

* Corresponding Author, Email: rajandce@iitg.ac.in

2Former Post-graduate Student, Department of Civil Engineering, IIT Guwahati, India

3PhD Research Scholar, Department of Civil Engineering, IIT Guwahati, India

doi: 10.1515/ijpeat-2016-0046

ABSTRACT

Several million tons of municipal solid wastes are generated worldwide, with plastic waste accounting for a major share every year. Plastic wastes are largely dumped in landfills or are burnt, leading to severe environmental concerns and challenges. The main aim of this study was to evaluate the effect of the incorporation of different waste plastics with variable sizes and dosages on the engineering properties of a stone matrix asphalt (SMA) mix. Three plastic types: polyethylene terephthalate (PET), high-density polyethylene (HDPE), and polypropylene (PP); two size ranges: 2.36-1.18 mm (coarser size) and 1.18-0.6 mm (finer size); and five dosages: 0, 4, 6 8 and 10% by weight of asphalt binder, were chosen for utilization in SMA mixes with a viscosity grade 40 (VG 40) asphalt binder. Shredded waste PET, HDPE, and PP in their respective sizes were incorporated in the SMA mix through the dry process, which involves the addition of shredded plastics to heated aggregates followed by mixing with hot asphalt binder. SMA mixes fabricated with waste plastics were evaluated for mechanical strength through Marshall stability and resistance against raveling through Cantabro abrasion loss tests. With the addition of waste plastic, SMA mixes yielded better performance compared to control mixes without plastics. The size of plastics showed a prominent effect on the performance of SMA mix characteristics. The addition of plastics showed improved strength and adhesion/abrasion loss characteristics for plastic dosage up to 6% for the coarser size, while up to 10% dosage for the finer size. Among plastic types, HDPE performed better than PP and PET in terms of strength and adhesion properties when used in coarser size, while PET yielded the best performance when used in finer sizes. The findings of the study indicate that both size and type of waste plastic play an important role in the performance of SMA mixes.

Keywords: stone matrix asphalt, plastic waste, polypropylene, high-density polyethylene, polyethylene terephthalate, Marshall stability, abrasion loss

INTRODUCTION

The rising quantities of waste materials is one of the most serious environmental challenges for the world. One of the most abundant and visible waste types is the plastic waste. It has been estimated that the world generated 7.8 billion tonnes of plastic by 2015, with an annual plastic production rate reaching 381 million tonnes in 2015 (Ritchie and Roser 2018). Globally, one million plastic drinking bottles are purchased every minute, while 5 trillion single-use plastic bags are used worldwide annually (UNEP 2018). According to Central Pollution Control Board (CPCB), around 3.47 million tons per annum of plastic waste was generated in 2019-20 in India (CPCB 2021). In the European Union, 41% of plastic garbage was incinerated, and around 30% of plastic waste was recycled; however, the United States Environmental Protection Agency (USEPA) reported that just 8.4% of plastic waste was recycled in the United States, and 75.8% was landfilled (Filho et al. 2019; USEPA 2019). Packaging was the most common use of primary plastics in the industrial sector in 2015 with around 42% share of total plastics usage, mostly composed of low-density polyethylene (LDPE), high-density polyethylene (HDPE), polypropylene (PP), and polyethylene terephthalate (PET) (Geyer et al. 2017).

Waste plastic imparts an anthropogenic impact on the environment and people, adversely affecting the ecosystem and health. The most straightforward approach to minimize waste plastic would be to limit its use. For instance, a proposed notion can be prohibiting single-use plastics in an effort to accomplish zero plastic waste generation (Wu and Montalvo 2021). However, this prohibition would be difficult to implement and enforce, and alternative solutions to the plastic waste problem must be considered. One such alternative route includes repurposing waste plastics in civil engineering infrastructure, such as the construction of flexible pavements made of asphalt mixes (Ahmadinia et al. 2011; Choudhary et al. 2018; Wu and Montalvo 2021). One of the majorly used asphalt mix types is stone matrix asphalt (SMA) which has a gap-graded aggregate skeleton with high coarse aggregate proportion, filler, and asphalt binder. SMA exhibits improved resistance to rutting and cracking, resulting in more durable road surfaces. Many studies have attempted utilizing various types of waste polymers and plastics in SMA mixes, and reported enhancement in the performance (Ahmadinia et al. 2011; Sarang et al. 2016; Modarres and Hamedi 2018; Lugeiyamu et al. 2021), but there is still a gap in thoroughly comprehending the efficacy of SMA mixes comprised of various types, dosages, and sizes of recycled plastics. The mechanical properties of the plastics-incorporated SMA mixes can be influenced by plastic related variables such as plastic type, content, and size. Therefore, a detailed study is needed that evaluates the effect of these variables on the properties of SMA mixes.

The main objective of this study is to investigate the effect of different types (PP, HDPE, PET), sizes (2.36 to 1.18 mm and 1.18 to 0.60 mm), and dosages (0, 4, 6, 8, and 10% by weight of binder) of recycled plastics on the performance of SMA mixes. The mechanical strength and resistance to raveling of the resulting SMA mixes were assessed using Marshall stability and Cantabro abrasion loss tests, respectively, and the results were compared to a control SMA mix (without plastic). The overview of the study is illustrated in Figure 1.

MATERIALS AND METHODOLOGY

Materials

A viscosity grade-40 (VG-40) bituminous binder and crushed natural stone aggregates obtained from a nearby local stone crusher were used for SMA mix preparation. The physical properties of the VG-40 binder which met the specified requirements in the Indian standard IS 73 (2013) are presented in Table 1. The physio-mechanical properties of the aggregates used in the study are shown in Table 2. The selected SMA aggregate gradation (mid-gradation of 13 mm SMA) and the corresponding to the upper and lower limits specified by Indian Roads Congress specification IRC SP 079 (2008) is shown in Figure 2. SMA mixes are prone to binder draindown during their mixing, hauling, and laying operations, and so to counteract the downward migration of bituminous binder under gravity at elevated temperatures, cellulose fiber at a dosage of 0.3% by weight of the mix was used in the study.

Figure 1 Experimental flow chart

Table 1 Physical properties of the VG-40 binder used in the study

| Property | Standards | Requirement | Results |
| --- | --- | --- | --- |
| Penetration at 25°C, 100 g, 5 s, 0.1 mm | IS 1203 | Min. 35 | 35 |
| Absolute viscosity at 60°C, Poises | IS 1206 (Part 2) | 3200-4800 | 4664 |
| Kinematic viscosity at 135°C, cSt | IS 1206 (Part 3) | Min. 400 | 765 |
| Flash point (Cleveland open cup), °C | IS 1448 [P: 69] | Min. 220 | >220 |
| Solubility in trichloroethylene, percent | IS 1216 | Min. 99.0 | >99 |
| Softening point (R&B), °C | IS 1205 | Min. 50 | 62.3 |
| Tests on rolling thin film oven (RTFO) residue |  |  |  |
| Viscosity ratio at 60°C | IS 1206 (Part 2) | Max. 4.0 | 2.15 |
| Ductility at 25°C, cm | IS 1208 | Min. 25 | >100 |

Table 2 Physio-mechanical properties of aggregates used in the study

| Property | Methodology | Test method | Specification | Test results |
| --- | --- | --- | --- | --- |
| Particle shape | Combined FI & EI | IS: 2386 Part I | Max. 30% | 16.80% |
| Strength | Los Angeles Abrasion | IS: 2386 Part IV | Max. 25% | 22.60% |
| Water absorption | Water absorption | IS: 2386 Part III | Max. 2% | 0.65% |
| Durability | Sodium Sulphate | IS: 2386 Part V | Max. 12% | 6.7% |

Figure 2 SMA gradation selected for the study

Three types of waste plastics: PP, HDPE, and PET, were collected from the Central Institute of Petrochemicals Engineering & Technology (CIPET) situated in Guwahati (Assam, India). At CIPET waste plastics were first segregated according to their type and then shredded to a size smaller than 4.75 mm using a mechanical shredder. Waste plastic corresponding to the size range of 2.36–1.18 mm and 1.18–0.60 mm was used in this study. The steps involved in the collection of the waste plastic to the procurement in desired sizes after shredding are illustrated in Figure 3.

Figure 3 (a) collection of waste plastic; (b) shredding of waste plastic; (c) shredded waste plastic; (d) storage of shredded plastic in bags; (e) PET of size 2.36-1.18 mm and 1.18-0.60 mm; (f) HDPE of size 2.36-1.18 mm and 1.18-0.60 mm; (g) PP of size 2.36-1.18 mm and 1.18-0.60 mm

Preparation of SMA Mix Specimens

For the fabrication of 100 mm diameter SMA mix specimens, the mixing operation was conducted at the mixing temperature of 165-170°C and compacted at 155-160°C employing 75 number of Marshal hammer blows on each face. The optimum binder content (OBC) of the control SMA mix fabricated with VG-40 was determined using the protocols stipulated in Indian standard IRC SP 079 (2008). The OBC of the control mix was found to be 6.4% by weight of the mix and met all the requirements stipulated in IRC SP 079 (2008).  The observed values for mix volumetric parameters for SMA mix prepared at 6.4% binder content are as follows: air void content = 4%; voids in mineral aggregate (VMA) = 17.4%; coarse aggregate voids in the mix (VCAmix): 31.9%; coarse aggregate voids in dry-rodded condition (VCAdrc): 41.2%; binder draindown: 0.19%.

The same OBC of 6.4% was also used as the binder content for all plastic added mixes to avoid binder content as a variable. For the preparation of plastic added SMA mixes, the dry process of mix modification was employed where shredded plastics were added to the heated aggregate at the desired dosages (4, 6, 8, and % by weight of binder) and mixed thoroughly. The prepared mixes were then compacted into 100 mm diameter specimens and satisfied the volumetric requirements of IRC SP 079 (2008).

Test Methodology

Marshall Stability

Marshall stability test was conducted on SMA specimens as per ASTM D6927 (2015). Specimens were first conditioned at 60 °C in a water bath for 30-40 minutes before measurement of Marshall stability. After the conditioning period, specimens were removed, patted with a towel to remove excess water, and quickly placed in the Marshall testing head. Loading ram was brought into contact with the testing head. Load at the rate of 50.8 mm/minute (2 inches/minute) was applied until the maximum load was reached. The maximum load to failure was reported as the stability.

Cantabro Abrasion Loss

In order to evaluate the effect of plastic type, content and size on the ravelling potential of SMA mixes, the Cantabro abrasion loss test was done. Under this test the compacted SMA mix samples were subjected to abrasion in the Los Angeles abrasion machine (without steel balls). The change in weight of the specimen before and after the test is a measure of the durability/abrasion resistance of SMA mix specimens. The initial weight of each SMA mix sample was noted and placed in the abrasion drum, which was then set for 300 revolutions at a rate of 30 revolutions per minute. The difference between the readings observed before and after the rotations was referred to as the Cantabro abrasion loss and was calculated in percentage using equation (1):

|  |  | (1) |
| --- | --- | --- |

where, W1 = Initial sample weight, g; W2 = Final sample weight, g

RESULTS AND DISCUSSION

Marshall Stability

Effect of Plastic Type

Figure 4 illustrates the Marshall stability results of SMA mixes fabricated with PET, HDPE, and PP at dosages of 0%, 4%, 6%, 8%, and 10% by weight of asphalt binder using the dry process. As depicted in Figure 4(a), after adding PET, HDPE, and PP of coarser size (2.36 mm-1.18 mm) the stability value increased for plastic content up to 6% and thereafter witnessed a reduction in the stability. On the other hand, for finer size (1.18 mm-0.60 mm) of PET, HDPE, and PP, the stability value increased until 10% dosage of plastic in SMA mixes, as shown in Figure 4(b). It is noteworthy that the Marshall stability was higher in comparison to the control mix for all the dosages when the finer size of plastics was used in SMA mixes. When compared for variation of Marshall stability among plastic types, HDPE performed best, followed by PP and PET. A higher content of coarser sizes of waste plastic may lead to improper mixing or a less homogeneous mix, resulting in a decrease in Marshall stability values after a dosage of 6%. This challenge was not observed with finer sizes, and the Marshall stability values increased with an increase in plastic dosage.

(a)

(b)

Figure 4 Variation of Marshall stability of SMA mixes for plastic types at different sizes: (a) 2.36-1.18 mm; (b) 1.18-0.60 mm

(a)

(b)

(c)

Figure 5 Variation of Marshall stability of SMA mixes with different plastic types: (a) PP; (b) HDPE; (c) PET

Effect of Plastic Size

Figure 5 shows the Marshall stability results of SMA mixes in terms of coarse (2.36-1.18 mm) and fine (1.18-0.60 mm) plastic sizes. Figures 5 presents that SMA mixes prepared with the coarser size of waste plastic, in general, showed a better performance up to 6 to 8% of dosage (it was 8% for PP and HDPE, and 6% for PET) when compared with the finer fractions. However, when the content increased by more than 8%, SMA mixes with finer sizes showed a higher Marshall stability than coarser mixes, indicating that both size and type play an important role in the optimum dosage of a plastic type in terms of Marshall stability strength.

(a)

(b)

Figure 6 Variation of Cantabro abrasion loss of SMA mixes for plastic types at different sizes: (a) 2.36-1.18 mm; (b) 1.18-0.60 mm

Cantabro Abrasion Loss

Effect of Plastic Type

Cantabro abrasion loss results of all the SMA mixes is shown in Figure 6. Cantabro abrasion loss helps to evaluate the binder's adhesion with aggregates and the cohesion of the mix. As shown in Figure 6(a), the abrasion loss value for plastic content reduced up to 6% after adding plastics of coarser size (2.36 mm-1.18 mm), but then increased for plastic dosage of 8% and 10%. In contrast, the abrasion loss for SMA mixes with finer size (1.18 mm-0.60 mm) of PET, HDPE, and PP shows a consistent decrease up to 10% plastic dosage used, as depicted in Figure 6(b). It was worth noting that when finer plastics were utilized in SMA mixes, abrasion loss significantly reduced in comparison to the control mix for all dosages. When it came to abrasion loss variation among plastic types, HDPE outperformed PP and PET. It can be deduced that the finer sizes showed a higher impact on the adhesion of the bituminous binder with the aggregates.

Effect of Plastic Size

Figure 7 depicts the abrasion loss of SMA mixes in terms of different sizes and dosages of PP, HDPE, and PET plastics. There was no definite trend observed with plastic size for the dosage of a particular plastic type. Figure 7(a) depicts that SMA mixes prepared with the finer size of PP showed lower abrasion loss than coarser size up to plastic content of 10%, while the abrasion loss had lower results in coarser size than finer in SMA mixes fabricated with HDPE and PET for dosages up to 6%. When dosages were 8% and 10%, SMA mix specimens generated with finer HDPE and PET had reduced abrasion loss than coarser size. However, the abrasion loss of all the SMA mixes with plastic waste was lower than the control mixes, but a definite trend was not observed with the increase in waste plastic dosage. This can be attributed to the combined effect of the general increase in strength of mix with plastic waste incorporation but a reduction in the adhesion of the aggregate and binder with an increase in dosage, which can depend on chemical characteristics of the plastic waste. In terms of abrasion loss, SMA mix specimens with HDPE of coarser size performed better than mixes prepared with the coarser size of PP and PET, while PET performed better than PP and HDPE among SMA mixes with the finer size of plastic.

(a)

(b)

(c)

Figure 7 Variation of Cantabro abrasion loss of SMA mixes with different plastic types: (a) PP; (b) HDPE; (c) PET

CONCLUSIONS

The main aim of this study is to investigate the properties of SMA mixes fabricated with varying types, sizes, and dosages. To see the influence of plastic types (PP, HDPE, PET) sizes (2.36-1.18 mm and 1.18-0.60 mm) and dosages (0%, 4%, 6%, 8%, and 10% by weight of asphalt binder) on the performance of SMA mixes, the Marshall stability test, and Cantabro abrasion loss test, was used. Based on the experimental data presented and discussed in this study, the following major conclusions are drawn.:

- The addition of plastic to SMA mixes improved the Marshall stability and resistance against raveling evaluated through Cantabro abrasion loss when compared to mixes without waste plastic.

- Among plastic types, HDPE performed better than PP and PET in terms of Marshall stability for both coarser and finer sizes.

- The size of plastics thus showed a prominent effect on the performance of SMA mix characteristics. The coarser size (2.36 mm-1.18 mm) of PET, HDPE, and PP showed increased Marshall stability value for plastic content up to 6% and thereafter witnessed a reduction in the stability. On the other hand, for finer size (1.18 mm-0.60 mm) of PET, HDPE, and PP, the stability value increased until 10% dosage of plastic in SMA mixes.

- SMA mixes prepared with the finer size of PP showed lower abrasion loss than coarser size up to plastic content of 10%, while the abrasion loss had lower results in coarser size than finer in SMA mixes fabricated with HDPE and PET for dosages up to 6%.

- Both size and type of waste plastic play an important role in the strength and adhesion/abrasion loss characteristics of SMA when used in variable dosages.

REFERENCES

Ahmadinia, E., Zargar, M., Karim, M. R., Abdelaziz, M., and Shafigh, P. (2011). Using waste plastic bottles as additive for stone mastic asphalt. Materials and Design, 32(10), 4844-4849.

ASTM D6927. (2015). Standard test method for Marshall stability and flow of bituminous mixtures. ASTM International, West Conshohocken, PA.

Choudhary, R., Kumar, A., and Murkute, K. (2018). Properties of waste polyethylene terephthalate (PET) modified asphalt mixes: dependence on PET size, PET content, and mixing process. Periodica Polytechnica Civil Engineering, 62(3), 685-693.

CPCB. (2021). Annual report 2019-20 on implementation of plastic waste management rules, 2016. Accessed from: https://cpcb.nic.in/uploads/plasticwaste/Annual_Report_2019-20_PWM.pdf

Geyer, R., Jambeck, J. R., and Law, K. L. (2017). Production, use, and fate of all plastics ever made. Science advances, 3(7), e1700782.

IRC SP 079. (2008). Tentative specifications for stone matrix asphalt. Indian Road Congress, New Delhi.

IS 73. (2013). Paving bitumen specification. Bureau of Indian Standards, New Delhi.

Lugeiyamu, L., Kunlin, M., Mensahn, E. S., and Faraz, A. (2021). Utilization of waste polyethylene terephthalate (PET) as partial replacement of bitumen in stone mastic asphalt. Construction and Building Materials, 309, 125176.

Modarres, A., and Hamedi, H. (2014). Effect of waste plastic bottles on the stiffness and fatigue properties of modified asphalt mixes. Materials & Design, 61, 8-15.

Ritchie, H. and Roser, M. (2018). Plastic pollution. Accessed from: https://ourworldindata.org/plastic-pollution#citation

Sarang, G., Lekha, B. M., Krishna, G., and Ravi Shankar, A. U. (2016). Comparison of Stone Matrix Asphalt mixtures with polymer-modified bitumen and shredded waste plastics. Road Materials and Pavement Design, 17(4), 933-945.

UNEP. 2018. Our planet is drowning in plastic pollution—it's time for change! Accessed from: https://www.unep.org/interactive/beat-plastic-pollution/

Wu, S., and Montalvo, L. (2021). Repurposing waste plastics into cleaner asphalt pavement materials: A critical literature review. Journal of Cleaner Production, 280, 124355.

CLASSIFICATION OF UNDERGROUND CAVITY BASED ON GEOMETRIC PARAMETERS FROM GROUND PENETRATING RADAR DATA

Carlo Elipse1: PhD Candidate, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 6499 4501, Email: carloelipse@gmail.com

Julius Marvin Flores2: Research and Development Department Manager, IRIS Technology, Seoul, South Korea, Tel: +82 10 2168 1434, Email: juliusmarvinflores@gmail.com

Van Phuc Tran2: PMS Department Team Leader, IRIS Technology, Seoul, South Korea, Tel: +82 10 4964 2168, Email:

vanphuctran053@gmail.com

Son Dong Nguyen1: PhD Candidate, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 8461 3504, Email: nguyensondong88@gmail.com

Hyun Jong Lee1*: Professor, Department of Civil and Environmental Engineering, Sejong University, Seoul, South Korea, Tel: +82 10 5360 2419, Email: hlee@sejong.ac.kr

doi: 10.1515/ijpeat-2016-0047

ABSTRACT

In Seoul, multiple occurrences of sudden collapse of pavement due to underground cavity led the city government to perform thorough investigation on city roads in which GPR survey was conducted. Using the GPR data from the survey, deep learning, which has been widely used in different fields of application, was utilized to detect cavity since it is evidently superior in terms of speed compared to typical method wherein experts manually inspect the GPR data. However, there is still a huge number of non-cavity locations misclassified as cavity. Therefore, there is a need for additional filter to eliminate the misclassified non-cavity locations. In this study, a method that utilizes parametric criteria based on the geometry of hyperbolic signals from GPR data of cavity and non-cavity locations is proposed to further discard non-cavity locations included in the cavity candidates from deep learning method results. A database of verified cavity and non-cavity locations were generated from the GPR survey all around South Korea. Each location’s B-scan and D-scan images were investigated, and the height, width, and length of the hyperbolic signal were measured. It was found that the ratio of B-scan and D-scan hyperbolas’ height, width, and length can be used to further distinguish cavity from non-cavity locations. The proposed classification method is validated using additional surveyed locations and the results showed that the number of misclassified locations in the cavity candidate locations significantly decreased.

Keywords: Ground Penetrating Radar; Underground Cavity

Introduction

Sinkholes, or sudden collapse of pavement, have been a pressing issue in Seoul for the past years. Multiple occurrences of pavement subsidence and road collapse have been reported around Seoul and a particular sinkhole in the underpass of Seokchon Lake with a width of 2.5 m, length of 8 m, and depth of 10 m caught the attention of the citizens (ABC News, 2015; Rhee et al., 2021). These phenomena not only pose massive traffic interruptions but also accidents that may harm the road users and pedestrians. As a response from the city government, an underground investigation was conducted on the roads of Seoul city by using a non-destructive testing equipment called the ground penetrating radar (GPR).

Basically, GPR is a non-invasive testing equipment that utilizes electromagnetic wave to visualize the underground layers beneath the surface. GPR technology has been used in various fields of science from archeology to engineering (Annan, 2002). In pavement engineering, GPR has been utilized in a wide range of purposes such as layer thickness estimation (Zhao & Al-Qadi, 2016), density and air void content determination (Al-Qadi et al., 2011), and underground anomaly detection and classification (Kang et al., 2019; Khudoyarov et al., 2020; Kim et al., 2019). In the case of Seoul city, GPR was used to detect underground cavity that has a high probability of becoming a massive sinkhole. Early detection of these underground cavities and rehabilitating it before expanding to a critical size prevent accidents that may occur due to sudden collapse of pavement.

Generally, underground cavities form a hyperbola signal in the cross-sectional radargram image generated from the GPR survey data. The conventional method of detecting underground cavity is by manually inspecting all the images that are produced from the GPR survey data. In this method, an expert with immense experience in inspecting GPR data is required. Since other underground objects such as pipes and other buried materials also create a hyperbola signal, exposure to hyperbola signals produced by different sources is needed to successfully differentiate one from the other. Thus, determination of underground cavity using visual inspection is highly subjective based on the expert’s experience. Moreover, it is very time-consuming given a huge amount of data to investigate. Therefore, researchers sought a faster method that can automatically detect the hyperbola signals and classify the underground object that created the signal.

Several researchers conducted studies to develop different methods to automatically detect and identify underground cavity from GPR survey data. Pattern recognition based on classical Hough transform (Capineri et al., 1998) and phase change of the signal (Falorni et al., 2004) were proposed. However, these methods consider only the hyperbola patterns specific to a single underground object, thus, lack the capability to classify the source of the hyperbola signal. Moreover, clutters and noises in the 3D GPR data caused by electromagnetic signals coming from other devices highly affect the detection results. In the past decade, deep learning techniques have gained its popularity in the field of object detection and classification, leading researchers to explore its other applications such as identification of underground object from GPR survey data. Mazurkiewicz et al. (2016) and Zhang et al (2016) both developed a simple artificial neural network to characterize underground object from GPR data. Moreover, other studies utilized convolutional neural network with varying input extracted from the GPR survey data. Kim et al. (2020) used the 2D images generated from the GPR data as input to train a CNN model, meanwhile, Khudoyarov et al. (2020) utilized 3D GPR block data, called a voxel. Although the mentioned approaches yielded promising results, the database used to train these deep learning models are either a full synthetic database generated by a finite-difference time-domain simulator or a limited number of field survey data, which lacks the diversity needed to successfully produce a trained model that is generally applicable for various actual field scenarios.

Recently, Tran (2020) developed a deep learning neural network with RetinaNet as the main  framework. The database compiled to train the model is composed of almost 10,000 2D images generated from 3D GPR data collected from field surveys all around South Korea. Moreover, the training database includes four different classes: cavity, pipe, manhole, and other underground objects that produce hyperbola signal such as large aggregates and buried materials, collectively called as non-cavity. The trained model was validated on a new set of survey data and yielded a total of 23 candidate locations. After coring the candidate locations, 12 out of 23 locations were verified as true cavity, resulting to a total accuracy of 52%. Meanwhile, the other 11 locations were non-cavities misclassified as cavities. Therefore, an additional process is needed to eliminate the misclassified non-cavity locations in the cavity candidates while preserving the true cavity locations.

In this study, a supplemental classification method that filters the candidate locations from RetinaNet-based deep learning neural network results based on the geometric features of the hyperbola signal was proposed. Using a compiled database of cavity and non-cavity locations, the hyperbola signals in the 2D images generated from the GPR data were measured and the ratio of the height, width, and length of the hyperbola signals in B-scan and D-scan images were calculated. Thresholds are determined to effectively isolate the cavity locations from the non-cavity locations. A classification method was introduced with parametric criteria based on the ratios of the geometric features of the hyperbola signal and its respective thresholds. Finally, validation was conducted to the proposed classification method by applying it to a new set of field survey data.

Background of the Study

Basic Principles of GPR

GPR, as mentioned, is a non-destructive testing device that mainly relies on the concept of electromagnetic waves. Basically, it is composed of four components: central unit, transmitting antenna or the transmitter, receiving antenna or the receiver, and the computer (Persico, 2014). The central unit of the GPR is responsible for the generation of the electromagnetic signal that is then emitted by the transmitter to the ground. As the electromagnetic signal hits an interface of two materials with different electromagnetic properties, part of the electromagnetic signal pass through the material below the interface while the other part of the electromagnetic signal reflects back to the surface and is then recognized by the receiver. The receiver sends to the computer the reflected electromagnetic signal’s amplitude and travel time from the transmitter to the receiver. Finally, the computer interprets and stores the information. As the GPR moves, the process from generating the electromagnetic wave to saving the information of the reflected signal repeats for every distance interval, called the trigger space, and the compiled information for the whole span of survey is the GPR data. Figure 1 illustrates the process of obtaining the GPR data from the central unit to the computer.

Figure 1. GPR data acquisition process from the central unit to the computer.

3D GPR data visualization

At an instantaneous point, a single array of GPR data can be obtained and can be plotted in amplitude of the reflected electromagnetic signal as the x-axis and its corresponding two-way travel time as the y-axis. This single array of GPR data is commonly known as A-scan (Daniels, 2004). In a GPR survey, numerous A-scan data can be obtained as the GPR device moves from one point to another and the group of A-scan data along the distance of the GPR survey is called the B-scan. In other words, B-scan is the cross-sectional view of the underground along the longitudinal direction of the road. B-scan can be plotted using the distance of the survey and the two-way travel time as the axes.

In a multichannel type of GPR, several transmitters and receivers are contained in the device and these transmitters and receivers can be paired up to each other to survey multiple B-scans, making the GPR data three-dimensional (3D). Each pair of transmitter and receiver is called a channel and channels are arranged perpendicular to the direction of the survey. In a specific point along the survey distance, A-scan data from all the channels can be obtained and this two-dimensional (2D) array of data is called the D-scan. Basically, the D-scan is the cross-sectional view along the transverse direction or perpendicular to the B-scan. D-scan can be represented in a plot with channels and two-way travel time as the axes.

Furthermore, in a 3D GPR data, a plane of data with the same two-way travel can also be obtained with distance of survey and channels as the axes. This 2D plane of data extracted from the 3D GPR data is called the C-scan. Generally, C-scan is the top view of the underground layer at a certain two-way travel time, making it perpendicular to the B-scan and D-scan. In other studies, C-scan is referred as the plane view at a specific depth since two-way travel can be converted to underground depth using the velocity of the electromagnetic signal at a certain medium.

From the B-scan, C-scan, and D-scan data, 2D images can be generated by transforming the amplitude of the reflected electromagnetic signal into pixel values. These images are utilized to investigate the underground layer’s profile as well as the objects buried underneath the surface. To have a better visualization of 3D GPR data, figure 2 illustrates A-scan, B-scan, C-scan, and D-scan in a 3D space, wherein x-axis is the distance traveled in the GPR survey, y-axis is the channel, and z-axis is the two-way travel time of the electromagnetic signal.

Figure 2. Visual representation of a 3D GPR data.

Cavity’s 3D GPR signal

Based on numerous studies (Kang et al., 2019; Khudoyarov et al., 2020; Kim et al., 2019; Tran, 2020), underground cavities form a hyperbola signal in 3D GPR data. This important feature has been widely utilized by GPR experts for visual inspection, as well as developers of automatic underground object detectors. Figure 3 exhibits a sample of the B-scan, C-scan, and D-scan images produced from the 3D GPR data of underground cavity.

Figure 3. Cavity 3D GPR images.

As seen in the figure, underground cavity produces a hyperbola signal in B-scan image as well as in the D-scan image. While in the C-scan image, an elliptical shape can be seen.

Non-Cavity’s 3D GPR signal

Meanwhile, some underground object such as large aggregates and other buried materials also form a hyperbola signal in the 3D GPR data. These underground objects, commonly called as non-cavities, are usually misclassified as underground cavities due to its similarity in geometric features. Figure 4 shows a sample of the B-scan, C-scan, and D-scan images generated from 3D GPR data of non-cavity location.

Figure 4. Non-cavity 3D GPR images.

Notice that in the B-scan and D-scan images, non-cavity also forms a hyperbola signal. Moreover, in the C-scan image, a similar shape with cavity, which is elliptical, can be seen.

These similarities in the signals of cavity and non-cavity in B-scan, C-scan, and D-scan images make it difficult to distinguish from one another. Based on this, relying solely on the hyperbola signal’s appearance may not be sufficient to effectively distinguish the two different classes from each other. Thus, an additional filter that utilizes quantitative criteria, such as the geometric features of the hyperbola signals, should be developed to classify cavity and non-cavity locations more accurately.

Determination of Classification Criteria

As concluded in the previous observations, quantitative criteria from the geometric measurements of the hyperbola signal should be determined to be able to further distinguish the cavity locations and discard the misclassified non-cavities. In order to determine the criteria to be used for the classification, data collection of 3D GPR data from numerous locations was conducted to compile a database that was then used for the parametric investigation.

Data Collection

The process flow for the data collection is shown in figure 5. Firstly, 3D GPR data were obtained by conducting surveys of the city roads and national highways in South Korea. After collecting the data, the RetinaNet-based deep learning neural network was applied to the 3D GPR data to detect and classify locations of hyperbola signals with a high probability of being an underground cavity, called the candidate locations. The candidate locations were accurately situated in the map based on the distance information in the 3D GPR data and were marked in the actual field. Each candidate location was investigated using the handy GPR to verify the exact location of the hyperbola signal and initially inspect if the detected hyperbola signal is produced by underground cavity. Several directions that intersect at the marked location were scanned to thoroughly inspect the underground object that causes the hyperbola signal. After inspecting the scanned images from the handy GPR, locations that appeared as pipe or any underground object other than cavity were discarded from the candidate locations and were not continued to be validated using borehole imaging. Meanwhile, locations with a strong hyperbola signal and high probability of being an underground cavity based on the handy GPR inspection were cored until the top of the underground object was reached. A borehole imaging device was inserted in the cored pavement and was lowered to the bottom of the borehole. As the camera was lowered, the device recorded the inside of the borehole while measuring the depth. The borehole image captured by the device was examined to validate the true classification of the candidate location.

In total, there are 854 locations in the database consisting of 282 cavity locations and 572 non-cavity locations. Based on the locations that were cored and verified using the borehole imaging, numerous non-cavity locations were wrongly detected by the deep learning neural network as cavity locations which was expected based on the observations from the hyperbola signals produced by cavity and non-cavity locations.

Figure 5. Process flow of data collection.

Parametric Investigation

After acquiring the database of cavity and non-cavity locations, investigation of the hyperbola signals in the B-scan and D-scan images were performed to identify which geometric parameters can be utilized as criteria for classification.

Previously, it was noticed that hyperbola signals are both present in the B-scan and D-scan images of cavity and non-cavity locations. However, differences in the geometrical parameters of the hyperbola, such as height, width, and length, in the B-scan and D-scan images were observed in the non-cavity signal when compared with that of the cavity signal. Cavity’s B-scan and D-scan hyperbola signal tend to have geometric parameters with almost similar measurements to each other since the typical propagation of underground cavity in longitudinal and transverse directions is almost at the same rate, starting from a round shape to an elliptical shape as it expands (Liu et al., 2020). Meanwhile, non-cavities are varying in shapes, thus, making the hyperbola signal’s geometric features in the B-scan and D-scan images different from each other. Therefore, the height, width, and length of the hyperbola signals in B-scan and D-scan images were measured in all the cavity and non-cavity locations included in the database. Figure 6 defines the geometric features measured in the hyperbola signal.

Figure 6. Height (HB and HD), width (WB and WD), and length (LB and LD) of the hyperbola signals.

To quantify the similarity or difference of the geometric features of the hyperbola signals in B-scan and D-scan, ratios of the corresponding geometric measurements were calculated, which is mathematically represented as follows:

(3)

(2)

(1)

H Ratio=HDHB

W Ratio=WDWB

L Ratio=LDLB

Where

H Ratio is the ratio of the hyperbola height

W Ratio is the ratio of the hyperbola width

L Ratio is the ratio of the hyperbola length

HB, HD are the height of the hyperbola at B-scan and D-scan

WB, WD are the width of the hyperbola at B-scan and D-scan

LB, LD are the length of the hyperbola at B-scan and D-scan

H Ratio, W Ratio, and L Ratio were calculated for all the locations in the database and the frequency distribution of the three parameters are shown in figure 7. It can be observed that most of the data for cavity locations are situated near 1, which means that the geometric features for B-scan and D-scan images were almost the same. Thus, these geometric parameters can be used to further isolate cavities and eliminate non-cavities in the list of candidate locations.

(a)

(b)

(c)

Figure 7. Frequency distribution for (a) H Ratio, (b) W Ratio, and (c) L Ratio.

Threshold Determination

From the geometric parameters identified in the investigation, upper and lower thresholds were determined that would effectively classify cavity and non-cavity locations. An optimization tool was used to identify the upper and lower thresholds of each parameter and table 1 summarizes the resulting values. These values were used in the proposed classification method.

Table 1. Threshold values for H Ratio, W Ratio, and L Ratio.

|  | H Ratio | W Ratio | L Ratio |
| --- | --- | --- | --- |
| Lower Threshold | 0.37 | 0.70 | 0.60 |
| Upper Threshold | 1.09 | 1.22 | 1.02 |

To determine how many criteria should be satisfied for the location to be classified as cavity, three different conditions were applied for the requirement of the classification: satisfy (1) at least one criteria, (2) at least two criteria, and (3) all three criteria. The accuracies for the three conditions were calculated and are summarized in figure 8.

Figure 8. Total classification accuracy for three different conditions.

As seen in the figure, satisfying at least two criteria gives the highest total accuracy, which is 90%, among the three conditions. By applying the condition of satisfying at least one criteria, the classification requirement is too lenient, thus, a lot of non-cavity locations will still be misclassified as cavity location, resulting to lower total accuracy. On the other hand, if all three criteria should be satisfied, the classification requirement is too strict, thus, several cavity locations will be classified as non-cavity and will be discarded from the candidate locations. Therefore, setting the condition to satisfy at least two criteria, most of the cavity locations will be retained as candidate while non-cavity locations will be discarded, giving the most reasonable accuracy for classification.

Proposed Classification Method

Based on the findings from the parametric investigation, a classification method utilizing the geometric features of the hyperbola signal in B-scan and D-scan images of candidate locations was proposed to further distinguish cavity locations from non-cavity locations. Figure 9 shows the process flow of the proposed classification method. In this method, ratios of the height (H Ratio), width (W Ratio), and length (L Ratio) of the hyperbola signals in B-scan and D-scan images were utilized as criteria. For a location to be classified as cavity, at least two out of the three parametric criteria should be satisfied.

Figure 9. Process flow of the proposed classification method.

The proposed classification method was applied to the database and the results are summarized in table 2. Based on the results, it was found that the total classification accuracy yields to 90%. As observed in the frequency distributions of the parameters in figure 7, there are overlaps between the area of cavity and non-cavity locations in the graphs. This means that some non-cavity locations are still included inside the lower and upper thresholds of the parametric criteria and some cavity locations are situated outside of the lower and upper thresholds of the parametric criteria, thus, wrongly classifying these locations. As a result, there are 34 cavity and 54 non-cavity locations that are misclassified by the proposed classification method.

Table 2. Summary of results of the proposed method using the database.

|  | Locations in the Database | Correct Classification | Wrong Classification | Total Accuracy |
| --- | --- | --- | --- | --- |
| Cavity Locations | 282 | 248 | 34 | 88% |
| Non-Cavity Locations | 572 | 518 | 54 | 91% |
| Total | 854 | 766 | 88 | 90% |

Validation of the Proposed Method

A new set of 3D GPR data were surveyed on Seoul city roads to validate the accuracy of the proposed method. Forty locations were chosen as candidate locations and among these locations, 20 locations are verified as cavity while the other 20 locations are non-cavity. The proposed classification method was applied to the candidate locations and the geometric parameters were calculated. After applying the classification method, it was found that the total accuracy for the validation of the proposed method is 88%. Among the 20 cavity locations, 19 locations were correctly classified as cavity. Moreover, 16 out of the 20 non-cavity locations were correctly classified as non-cavity, thus, can be eliminated using the proposed method. Table 3 summarizes the results for the validation of the proposed classification method.

Table 3. Validation results of the proposed method.

|  | Locations in the Database | Correct Classification | Wrong Classification | Total Accuracy |
| --- | --- | --- | --- | --- |
| Cavity Locations | 20 | 19 | 1 | 95% |
| Non-Cavity Locations | 20 | 16 | 4 | 80% |
| Total | 40 | 35 | 5 | 88% |

Summary and Conclusion

In this study, a classification method was developed based on the geometric parameters of the hyperbola signal from the 3D GPR data to reduce the number of candidate locations by eliminating misclassified non-cavity locations. Numerous GPR surveys were conducted all around South Korea to compile a database that includes 3D GPR hyperbola signals from cavity and non-cavity. Parametric investigation was performed in the hyperbola signals of the locations to identify the criteria to be used in the proposed classification method. Consequently, the proposed classification method was validated using additional dataset of locations from Seoul city. The results of the study are summarized as follows:

- Even though both cavity and non-cavity produce hyperbola shape in the 3D GPR data, geometric features of the hyperbola signal, such as height, width, and length, can still be used to further distinguish the underground object that creates the signal.

- Ratios of height (H Ratio), width (W Ratio), and length (L Ratio) of the hyperbola signal of B-scan and D-scan images were calculated for all the locations included in the database. Using an optimization tool, upper and lower threshold were determined can be applied to effectively isolate cavity locations from non-cavity locations. H Ratio, W Ratio, and L Ratio were considered as parametric criteria for the proposed classification method.

- For a hyperbola signal to be classified as cavity, at least two out of the three parametric criteria should be satisfied. Applying this to the database, the total classification accuracy resulted to 90%.

- Validation was conducted on the proposed classification method by applying it to a new set of locations. It was found that the total accuracy for classification using the validation dataset is 88%.

From these results, it can be concluded that the proposed classification method can be used to further eliminate misclassified non-cavity locations, thus, reducing the candidate locations from the RetinaNet-based deep learning neural network results.

References

ABC News. (2015, February 25). Sinkhole swallows two pedestrians in Seoul, pair survive fall with only minor injuries - ABC News. https://www.abc.net.au/news/2015-02-25/two-pedestrians-rescued-after-falling-into-a-sinkhole/6260590

Al-Qadi, I. L., Leng, Z., & Larkin, A. (2011). In-place hot mix asphalt density estimation using ground penetrating radar.

Annan, A. P. (2002). GPR-History,Trends and Future Developments. Subsurface Sensing Technologies and Applications, 3(4), 253–270.

Capineri, L., Grande, P., & Temple, J. A. G. (1998). Advanced image-processing technique for real-time interpretation of ground-penetrating radar images. International Journal of Imaging Systems and Technology, 9(1), 51–59. https://doi.org/10.1002/(SICI)1098-1098(1998)9:1<51::AID-IMA7>3.0.CO;2-Q

Daniels, D. J. (2004). Ground Penetrating Radar (Second Edi). IEE Press.

Elipse, C. (2021). Development of Deep Learning Neural Network for Underground Cavity Classification Based on Geometric and Numerical Parameters.

Falorni, P., Capineri, L., & Masotti, L. (2004). 3-D Radar imaging of buried utilities by features estimation of hyperbolic diffraction patterns in radar scans. Tenth International Conference on Ground Penetrating Radar, 403–406.

Kang, M. S., Kim, N., Im, S. B., Lee, J. J., & An, Y. K. (2019). 3D GPR image-based UcNet for enhancing underground cavity detectability. Remote Sensing, 11(21), 1–18. https://doi.org/10.3390/rs11212545

Khudoyarov, S., Kim, N., & Lee, J. J. (2020). Three-dimensional convolutional neural network–based underground object classification using three-dimensional ground penetrating radar data. Structural Health Monitoring, 19(6), 1884–1893. https://doi.org/10.1177/1475921720902700

Kim, N., Kim, K., An, Y. K., Lee, H. J., & Lee, J. J. (2020). Deep learning-based underground object detection for urban road pavement. International Journal of Pavement Engineering, 21(13), 1638–1650. https://doi.org/10.1080/10298436.2018.1559317

Kim, N., Kim, S., An, Y. K., & Lee, J. J. (2019). Triplanar Imaging of 3-D GPR Data for Deep-Learning-Based Underground Object Detection. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 12(11), 4446–4456. https://doi.org/10.1109/JSTARS.2019.2953505

Liu, T., Wang, H., Su, X., Li, K., Liu, S., & Guo, F. (2020). Instability Mechanism of Cavity-Bearing Formation under Tunnel Excavation Disturbance. Advances in Civil Engineering, 2020. https://doi.org/10.1155/2020/9038421

Mazurkiewicz, E., Tadeusiewicz, R., & Tomecka-Suchoń, S. (2016). Application of Neural Network Enhanced Ground-Penetrating Radar to Localization of Burial Sites. Applied Artificial Intelligence, 30(9), 844–860. https://doi.org/10.1080/08839514.2016.1274250

Persico, R. (2014). Introduction to Ground Penetrating Radar: Inverse Scattering and Data Processing. In Introduction to Ground Penetrating Radar: Inverse Scattering and Data Processing (First Edit). John Wiley & Sons, Inc. https://doi.org/10.1049/pbce115e_ch1

Rhee, J. Y., Park, K. T., Cho, J. W., & Lee, S. Y. (2021). A study of the application and the limitations of gpr investigation on underground survey of the Korean expressways. Remote Sensing, 13(9). https://doi.org/10.3390/rs13091805

Tran, V. P. (2020). Application of Deep Learning Algorithm for Pavement Object Detection Part 1: Asphalt Pavement Crack Detection Part 2: Cavity Detection. Sejong University.

Zhang, Y., Huston, D., & Xia, T. (2016). Underground object characterization based on neural networks for ground penetrating radar data. Nondestructive Characterization and Monitoring of Advanced Materials, Aerospace, and Civil Infrastructure 2016, 9804. https://doi.org/10.1117/12.2219345

Zhao, S., & Al-Qadi, I. L. (2016). Development of an analytic approach utilizing the extended common midpoint method to estimate asphalt pavement thickness with 3-D ground-penetrating radar. NDT and E International, 78, 29–36. https://doi.org/10.1016/j.ndteint.2015.11.005

ASPHALT REINFORCEMENT – A PROVEN ECONOMIC & ECOLOGICAL ASHPALT REHABILTIATION METHOD

Andreas Elsing

Dipl.-Ing., HUESKER Synthetic GmbH, Fabrikstrasse 13-15, Gescher, 48712, Germany, Tel: +49 2542 7010, elsing@huesker.de

Christoph Hessing

Dipl.-Ing., HUESKER Synthetic GmbH, Fabrikstrasse 13-15, Gescher, 48712, Germany, Tel: +49 2542 7010, hessing@huesker.de

Graham Horgan

Director, Huesker Limited, 1 Quay Business Centre, Warrington, WA2 8LT, UK

Tel: +49 1925 629393, grahamh@huesker.co.uk

doi: 10.1515/ijpeat-2016-0048

ABSTRACT

Asphalt reinforcement products manufactured using polyester fibres have successfully been applied in pavement rehabilitation for more than 45 years. Their performance has helped to increase maintenance periods, which provides a substantial financial and ecological benefit, in form of a reduced whole life costs, reduced use of finite resources and associated benefits (i.e., reduced traffic disruption).

The wider benefits of asphalt reinforcement and in particular those produced from polyester fibres are discussed. This paper identifies several key factors influencing the performance of asphalt reinforcement products and the performance of an asphalt reinforcement as a composite with asphalt and reinforcement considered together. Moreover, the sustainability benefits of asphalt reinforcement are discussed by assessing and quantifying the embodied carbon dioxide (ECO2) of road pavement construction and maintenance processes to provide a baseline against which these processes can be measured, evaluated and compared.

A simplified comparison is presented whereby a conventional road pavement maintenance project is compared with and without asphalt reinforcement and provides a good estimate of the carbon dioxide (CO2) savings for the rehabilitation method with asphalt reinforcement.  More than 60 % of the CO2 emissions generated during the entire construction phase are caused solely by the energy-intensive production of the asphalt mixes and the associated processes. This was the result of a detailed investigation of all CO2 emissions in a conventional road pavement maintenance project. In comparison a CO2 saving of 24.3 t CO2 per rehabilitated road kilometre was identified in a project where the cracked binder is being reinforced instead of replaced. The recyclability of this asphalt reinforcement is also discussed to establish if it supports the move to a more circular economy.

Keywords: Asphalt Reinforcement, Polyester, Embodied Carbon Dioxide (ECO2), Emission Savings, Recyclability

# INTRODUCTION

Asphalt reinforcement has been used worldwide for many years to delay or prevent reflective cracks in asphalt layers. Using asphalt reinforcement can clearly extend the fatigue life and therefore the maintenance intervals of rehabilitated asphalt pavements. This increase in pavement life does have the positive effect that not only the maintenance costs per year but also the amount of energy used for maintenance per year can be significantly reduced.

Environmental and climatic protection is gaining an ever increasing importance. Several road authorities have signed up to ambitions carbon emissions reduction plans aiming to significantly reduce emissions of both the construction and maintenance of their road network, recent commitments signed at Cop26 in Glasgow saw many countries signing up to net zero carbon emissions by 2050, (i.e. when the amount of carbon we add to the atmosphere is no more than the amount removed).

By assessing and quantifying the Embodied CO2 of road pavement construction and maintenance processes these provide a baseline against which these ambitious ‘net zero’ targets can be measured and evaluated. Several manufacturers are already providing Environmental Product Declarations (EPD’s) which aim to quantify the embodied CO2 emissions of their product on a ‘cradle to grave’ basis which allows a full assessment of the embodied CO2 for both the initial construction and over the full, service life of the infrastructure asset.

For a typical rehabilitation project the actual amount of embodied CO2 of the asphalt reinforcement itself is relatively small when compared to that of a typical 50 mm asphalt layer or other energy-intensive materials like concrete or reinforced concrete steel, however the sustainability benefits it can bring about by delaying or preventing the replacement of these energy-intensive primary materials can be significant.

An additional consideration is the reuse of recycled materials and whether these support the principals or of circular economy. Recycled Asphalt plainings (RAP) are now considered for reuse in new asphalt mixes. Investigations were undertaken to determine if the incorporation of HaTelit asphalt reinforcement in a pavement would have any detrimental effect on the recyclability of that pavement at the end of service life and if residual granules of the reinforcement would affect the new recycled asphalt mix design or performance. Moreover some manufacturers have started to use recycle polymers to produce asphalt reinforcement. Recycling of single use polyester bottles effectively produces polymer granulate which can be used to produce virgin quality fibres that are incorporated into asphalt reinforcement grids. The performance benefits of polyester-based asphalt reinforcement are subsequently discussed together with embodied CO2 of these combined solutions and how use of these material supports a ‘net zero’ strategy

# REFLECTIVE CRACKING AND ASPHALT REINFORCEMENT

Cracks appear in asphaltic pavements due to external forces, such as traffic loads and temperature variations. The temperature influence and the dynamic loading over time leads to the binder content in the asphalt becoming brittle. High stresses at the bottom of a pavement, from external dynamic loads, such as traffic, can cause cracks which propagate from the bottom to the top of a pavement (bottom-up cracking). Additionally underlying discontinuities in the pavement subgrade/base course can also reflect up

Figure 1: Simulated stress distribution with FE-Method, without (left) and with reinforcement (right)

through the overlying asphalt.

In order to delay the propagation of cracks into the asphalt layers an asphalt reinforcement geosynthetic can be installed. The reinforcement increases the resistance of the overlay against high tensile stresses and distributes them over a larger area, thereby reducing the risk of local overstressing which would result in cracks.

# WHY POLYESTER REINFORCEMENT?

The wider benefits of asphalt reinforcement and in particular those produced from polyester fibres was discussed previously (Elsing & Horgan, 2019).  This paper identified several key factors influencing the performance of asphalt reinforcement products and the performance of an asphalt reinforcement as a composite, with asphalt and reinforcement considered together.

- Bond Stiffness

- Bonding Strength: Mobilisation of tensile forces

- Thermal expansion coefficient

- Field testing

- Laboratory Testing

- Cyclic load and Fatigue testing

Typically, the testing procedures, to determine the tensile strength and elongation (e.g. ISO EN 10319:2015), of Asphalt reinforcements are undertaken in “air” and not embedded in asphalt. An asphalt reinforcement improves the stress-strain properties of an asphalt pavement by adding tensile strength into the asphalt system. However, other properties (other than the in air measured reinforcement stiffness) are influencing the performance of an asphalt reinforcement and that asphalt/reinforcement interface bond is an important influence factor in transferring tensile forces into reinforcement on the contact surface.

The performance of an asphalt reinforcement should, therefore, ideally be determined as a composite, with asphalt and reinforcement considered together. Many tests have been performed in the past decades to demonstrate the performance of asphalt reinforcement produced with high tenacity Polyester fibres. The aim of such tests was to replicate, as close as possible, the installed in-service composite behaviour in laboratory conditions, whilst also trying to qualify the important parameters which influence the behaviour of an asphalt reinforcement.

## Bond Stiffness

De Bondt (1999) published "Anti-Reflective Cracking Design of (Reinforced) Asphaltic Overlays", which was the last phase of the Ph.D. program and a 5 year research project at the Delft University of Technology. De Bondt determined the relevance and influence of different parameters on reflective cracking in asphalt overlays, and performed comparative investigations on different commercially available products in the market at that time.

It was found that one of the most important parameters is the bonding of the reinforcement to the asphalt, he defined as ‘bond stiffness’. De Bondt determined the equivalent bond stiffness in reinforcement pull-out tests on asphalt cores taken from a trial road section. Parts of the results are presented in Figure 2. De Bondt determined that the equivalent bond stiffness of a polyester reinforcement was by far the most effective of all products investigated. The importance of the bituminous coating for flexible grids has a significant influence. De Bondt found that in flexible grids like a polyester reinforcement the stresses were transmitted via direct adhesion between strands and asphalt – hence the coating plays a vital part to the ultimate performance.

Figure 2: Equivalent bond stiffness (ceq,rf in N/mm/mm² ) of different investigated products (De Bondt, 1999)

By using finite element models, De Bondt calculated the improvement factors for reinforcements based on material stiffness (EArf) and pull-out stiffness (ceq,rf). With a product stiffness of ~900 N/mm and a pull-out stiffness (ceq,rf) of about 9, the polyester reinforcement achieves an improvement factor of 3.5 in the simulation compared to a control section. (i.e. achieves a number of load cycles 3.5 times the number of cycles for the control section without reinforcement before reflective cracks reappear).

From this research it is evident, that a good bonding of the reinforcement to the asphalt is very important for the performance of an asphalt reinforcement. The combination of high reinforcement stiffness (Polyester) and high bond stiffness (Bitumen impregnation) creates a high improvement factor for the overlay life of an asphalt pavement.

## Bonding strength

To mobilize tensile forces in the reinforcement a good bonding between the asphalt layer and the integrated reinforcement is essential. Based on the German guideline ZTV Asphalt-StB 07/13 the shear force within the testing procedure according to Leutner should not be lower than 15.0 kN between the binder course and the surface layer.

Figure 3: Drill core with reinforcement (after testing)

Table 1 shows the results of drill cores tested at the University RWTH Aachen (2018). These exemplary results show that the bonding strength is not significantly influenced by a bitumen coated Polyester grid.

Table 1: Comparison of shear forces of unreinforced drill cores and HaTelit reinforced drill cores acc. to Leutners method

## Thermal expansion coefficient

High modulus polyester is a flexible raw material with a maximum tensile strain less than 12%. The coefficients of thermal expansion of polyester and asphalt (bitumen) are very similar (Table 2). This leads to very small internal stresses between the PET fibres and the surrounding asphalt (similar to reinforced concrete) and minimizes/avoids the phenomenon of progressing de-bonding and as result of repeated daily and seasonal thermal cycles. For this reason Polyester does not act as an extrinsic material in the reinforced asphalt layer

Table 2: Comparison of the thermal expansion coefficient

| Material combination | Thermal expansion coefficient | Ratio |
| --- | --- | --- |
| Concrete / Steel | 1.3 x 10-5 / 1.0 x 10-5 | ~ 1 : 1 |
| Asphalt / Polyester | 6.0 x 10-4 / 1.6 x 10-4 | ~ 1 : 4 |
| Asphalt / Fiberglass | 6.0 x 10-4 / 4.5 x 10-6 | ~ 1 : 130 |

## Field Tests: Federal Materials Testing Institute (EMPA 2017)

The aim of the research work was to obtain information on the effectiveness of different asphalt interlayers (Figure 4) and the corresponding service life of the pavement.  Surface characteristics of the raw material and their effect on the in-situ activated tensile strength were assessed.  Both laboratory and field tests were undertaken to assess the performance of different products and focus was given on the time dependent development of cracks and an accelerated loading cycles in the field to determine the number of pavement loading cycle until the development of reflective cracking.

Figure 4: Grid A: Glass fibre grid + SAMI; Grid B: Glass and carbon fibre grid; Grid C: Polyester Grid

Figure 5: Schematic view of Field test set and pavement thickness

Both laboratory and field tests showed that asphalt reinforcement systems can delay the development of cracks, thereby extending the life of asphalt pavements. Further, it was observed that asphalt reinforcements (systems B and C) are more effective if no SAMI (stress absorbing membrane interlayer) is used.

## Laboratory Testing: University of Texas

A study on geosynthetic-reinforced asphalt systems was published by Luming, (2018).  This research presents a study on various aspects relevant to geosynthetic-reinforced asphalts. Specifically, this thesis has been organized in three stand-alone sections: 
1) Literature Review; 2) Overlay Testing; 3) Shear Fatigue Testing.

Section II presents the experimental research that was conducted using overlay testing involving geosynthetic-reinforced asphalt specimens. The standard overlay test has been designed to evaluate crack propagation in asphalt concrete using a fatigue loading mechanism that induces tensile and shear stresses.  The experimental study presented in Section II adopted this test to evaluate the effectiveness of the different geosynthetics in retarding the reflective cracking from an old asphalt into a new overlay. The asphalt specimens were tested in the standard overlay test along with geosynthetic-reinforced asphalt specimens. In addition, an image acquisition system was used to track propagation of cracks during overlay tests, see Figure 6

Figure 6: Test set up for shear fatigue test

Four types of reinforcement were tested.  Each reinforcement had the same mesh size and the same coating to eliminate a couple of potential variables and to provide a better understanding of the influence of the raw material. Polyester (PET), Glass fibre and Polyvinylalcohol (PVA) were tested.

Table 3: Tested asphalt reinforcement from different raw materials

|  | Reinforcement 1 | Reinforcement 2 | Reinforcement 3 | Reinforcement 4 |
| --- | --- | --- | --- | --- |
| Raw material | PVA | PET | Glass | Glass |
| Tensile strength [kN/m] | 50 | 50 | 50 | 100 |

## Summary and Conclusions from Overlay Tests

Parts of the results are presented here, for full details the reader may refer to the full publication, Luming, (2018).  With the opening and closing of the simulated existing crack, the cross-crack initiates and propagates from the tip of the simulated crack towards the top surface of the asphalt concrete. Overall, the reinforced asphalt specimens showed better fatigue performance than the unreinforced asphalt concrete. The PVA and PET fibres are more compatible with the asphalt concrete than the glass fibre, thus they can better interact with the asphalt specimen at later fatigue life.

The normalized load of the PVA reinforcement at the end of phase 4 was the highest, indicating the best performance of this material in enhancing the shear resistance of the asphalt concrete over fatigue life. The PET reinforcement shows second best results, close to them of PVA. The performance of the glass fibre reinforced specimens was not as good as the polymer reinforced specimens in terms of retarding the load decline. This could be attributed to the varied compatibility of the reinforcement with the asphalt concrete. PVA and PET consist of polymer fibres which are more compatible with the asphalt concrete in the stiffness of the materials than the glass fibre.

## Laboratory Testing: Fatigue

The textbook definition of fatigue theory states that fatigue cracking initiates at the bottom of the flexible layer due to repeated and excessive loading, and it is associated with the tensile strains at the bottom of the HMA layer (Huang, 1993). The fatigue cracking in cracked pavements can be significantly delayed, by reducing the tensile strains at the bottom of a flexible asphalt layer (Figure 7).

According to the function of fatigue: Nf = k1 (1/εt)k2

Nf 	= allowable load repetitions of a pavement (until failure occurs)

k1 	= coefficient of fatigue

k2 	= exponent of the fatigue function

εt 	= elongation on the bottom of the asphalt layer [%]

Figure 7: Schematic view of the fatigue cracking mechanism in pavement cross section without and with reinforcement

From specialist literature the value mentioned for the coefficient factor of fatigue k1 is 2.0 x 10-12. For k2, the exponent of fatigue function is 5.0. At a vertical deformation of 0.5 mm during a loading cycle an elongation of 0.0001 % is measured below the asphalt layer. A small reduction in the elongation below the asphalt layer already has significant effects on the allowable loading cycles. Detailed figures are presented in Table 4.

Table 4: Calculated loading cycles until failure occurs

| Reduction of elongation | Elongation [εt] | Loading cycles [Nf] | Improvement Factor |
| --- | --- | --- | --- |
| Reference | ε= 0,000100% | 2.00 x 108 | - |
| -5% | ε= 0,000095% | 2.99 x 108 | 1.5 |
| -10% | ε= 0,000090% | 3.99 x 108 | 2.0 |
| -20% | ε= 0,000080% | 6.10 x 108 | 3.0 |

## Laboratory Testing: Cyclic loading and Deformation

Figure 8: Test set-up cyclic loading test (schematic)

In a diploma thesis by Höptner 2010 the benefits of asphalt reinforcement in road rehabilitation by using a modified rutting simulator have been investigated. The aim of this research was to analyse the influence of an asphalt reinforcement on reducing the deformation in pavements. The setup has been prepared according to realistic pavement design. The pre-cracked specimens have been located on an elastic rubber foundation which simulates the base course (Figure 8). The force has been applied by a rubber wheel. For the test set up a standard asphalt design has been chosen, with a 60mm binder course (AC 16 B S) and a 40mm surface course (SMA 8 S). The specimen was prepared in a roller sector compactor. In the first step the binder layer (including the simulated crack) was prepared

After preparing the binder course (including a simulated crack) specimens with and without reinforcement were produced. The reinforced specimen was impregnated with a bituminous emulsion (C67B4-OB) in accordance with the installation guideline of the producer of the asphalt reinforcement (Figure 9).

Figure 9: Pre cracked specimen with asphalt reinforcement

A force of 700N has been applied onto the specimen by a rolling rubber wheel, which is equivalent to a 10 tonne axle load. Two identical asphalt specimens have been produced, with, and without polyester reinforcement. The deformation from loading cycle 50,000 to the end of the testing at 60,000 loading cycles was, without reinforcement, 2.1mm. The measured deformation with reinforcement, only was 1.0mm. This results in a reduction of 50% in deformation. (Figure 10).

-50%

Figure 10: Deformation with and without Polyester reinforcement of an asphalt specimen between 50.000 and 60.000 loading cycles

## Laboratory Testing: Cyclic loading and crack propagation

A full description and the results of a testing program performed at the Aeronautics Technological Institute in Sao Paulo, Brazil, were published by Montestruque in 2004. In this research program which started in 1999, an asphalt wearing course was applied over an existing crack in a detailed series of tests (Figure 11).

Figure 11: Dynamic Testing at ATI (Brazil) - Bending Mode

Both the bending mode and the shear mode were investigated under dynamic fatigue loading conditions. The results confirmed that HaTelit considerably delays the penetration of cracks. Compared to the unreinforced samples, the HaTelit reinforced asphalt layers were subjected to up to over 5 times the number of dynamic load cycles before a crack reached the surface. The crack pattern clearly shows that the reinforcement absorbs the high tensile forces and distributes over a larger area.

# EMBODIED ENERGY (EE) AND EMBODIED CO2 (ECO2)

Since the 1980's sustainability has been used in the sense of human sustainability on planet Earth and this has resulted in the most widely quoted definition of sustainability and sustainable development, that of the Brundtland Commission of the United Nations:

"Sustainable development is development that meets the needs of the present without compromising the ability of future generations to meet their own needs." (Brundtland Commission, 1987)

In the context of the construction industry this does mean that different construction techniques and designs for a specific project are compared for their ECO2 as an indicator for their sustainability. As a matter of fact the ECO2 is only one criterion beside social and economic considerations. Recent commitments signed at Cop26 in Glasgow saw many countries signing up to net zero carbon emissions by 2050. By assessing and quantifying the embodied energy (EE) and embodied carbon dioxide (ECO2) for the materials used on site without considering the individual transport distances and their installation. The authors of this paper appreciate that this comparison is not in line with the typical "cradle to grave" approaches used in this field, but it has been previously shown that the following comparison is sufficiently detailed to compare the two construction techniques without compromising on the accuracy of the results.

## Data source

The ECO2 values ("Carbon Footprint") used in the following chapters are taken from the latest Inventory of Carbon & Energy (ICE) V2.0. The University of Bath has created the ICE embodied energy & embodied carbon database which is the freely available. The aim of this work is to create an inventory of embodied energy and carbon coefficients for building materials. The data base is structured into 34 main material groups (i.e. Aggregates, Aluminium, Asphalt, etc.).

## Examples of embodied CO2

The amount of embodied carbon dioxide per kg of material can vary significantly as can be seen in Table 5. The more processing and energy that is required to achieve the final product the higher is the ECO2.

Table 5: Examples of embodied carbon dioxide (ECO2) in construction materials (cradle to gate)

| Material | kg ECO2 / kg of material | Note |
| --- | --- | --- |
| Aggregate | 0.0052 | gravel or crushed rock |
| Aluminium | 9.16 | - |
| Asphalt | 0.076 | 6% binder content |
| Bitumen | 0.55 | - |
| Cement | 0.74 | UK weighted average |
| Concrete 16/20 | 0.10 | unreinforced |
| Reinforced Concrete RC 40/50 | 0.188 | high strength applications / precast |
| PVC General | 3.10 | - |
| HDPE | 1.93 | - |
| Steel | 1.46 | average UK recycled content |
| Steel | 2.89 | Virgin steel |
| Source: ICE Inventory of Carbon & Energy V2.0 |  |  |

Energy intensive processes like the production of cement are producing a high amount of CO2. Cement manufacturing releases CO2 in the atmosphere both directly when calcium carbonate is heated, producing lime and carbon dioxide, and also indirectly through the use of energy if its production involves the emission of CO2.

On the basis of a specific Environmental Product Declaration (EPD) the embodied CO2 for HaTelit C 40/17 eco asphalt reinforcement, made of recycled PET, has been assessed and externally verified as 1.05 kg ECO2/m² of material (3.88 kg ECO2/kg of material). This assessment is based on a cradle to grave approach and thus exceeds the system boundaries for the examples presented in in Table 5.

# COMPARISON OF EMBODIED CARBON DIOXIDE FOR REINFORCED AND UNREINFORCED ASPHALT OVERLAYS

The report "Sustainable geosystems in civil engineering applications" commissioned by the Waste and Resource Action Plan (WRAP, 2010) has analysed geosystems as alternatives to standard designs used by civil engineers.

Parallel to geosystems for ground engineering the report has identified that "Reinforcement of the asphaltic or bound layers can increase the life of the surface layers, again by contributing to a strengthening of the bound layers. Such strengthening increases their ability to resist cyclic fatigue, thermal stresses during extremes of winter and summer temperatures, as well as increasing resistance to near-surface crack propagation." (WRAP, 2010).  The report clearly identifies that asphalt reinforcements can extend pavement life by limiting reflective cracking and thus providing more sustainable pavements as a consequence. This paper aims to demonstrate the above referenced effect by comparing the ECO2 based on the material consumption per year of lifetime of two construction techniques. One construction technique is the conventional rehabilitation of cracked overlays by milling and repaving, the second is a rehabilitation using PET asphalt reinforcement in the same process.

## Basis for Calculation

The example chosen for this comparison is a typical rehabilitation project with 5,000 m² of cracked wearing course to be replaced. Although the project size does not have any effect on the relative saving of ECO2 it helps to give a better assessment for the saving potential.

Table 6: Basis for calculation

| Job size | 5,000 m² |
| --- | --- |
| Asphalt thickness to be replaced | 40 mm |
| Density of asphalt | 2,500 kg/m³ (compacted) |
| Bituminous emulsion (70%) | 0.3 kg/m² (unreinforced) |
| Bituminous emulsion (70%) | 1.0 kg/m² (reinforced) Note (a) |
| HaTelit asphalt reinforcement | 0.27 kg/m² (made of recycled PET) |
| Improvement factor - reinforced to unreinforced asphalt | 3 [-] Note (b) |
| Design life (unreinforced): | 4 years Note (c) |

Notes:

(a)	Required amount of bituminous emulsion for HaTelit asphalt reinforcement 	over 
 	milled surfaces acc. to manufacturer's recommendations.

(b)	The improvement factor of 3 for the life time of reinforced asphalt as compared to
	unreinforced asphalt has been selected on the lower side of the potential range of 
 	3 – 4 to account other potential failure mechanisms which makes rehabilitation
	necessary but are not related with reflective cracking.

(c)	The design life of the unreinforced asphalt overlay has been chosen as 4 years since a typical crack propagation rate of approx. 10 mm / year would result in cracks reaching the surface of the new overlay after 4 years. The crack propagation rate of approx. 10 mm / year is of course project specific and could vary.

## Comparative calculation of the embodied CO2 for reinforced and unreinforced asphalt overlays

In the comparison in Table 7 it can be seen that a conventional (unreinforced) rehabilitation method results in 7.72 kg embodied CO2 per m² for the materials used. The alternative design using a PET asphalt reinforcement results in 9.04 kg embodied CO2 per m² due to the additional asphalt reinforcement and a higher amount of bituminous emulsion. The comparison of the ECO2 for the rehabilitation project has to be put into relation with the design life. The design life for the unreinforced overlay is set to 4 years until first cracking is likely to have reached the surface again. The reinforced overlay on the other side would last at least 3 times longer, i.e. 12 years.

Table 7: Comparative calculation of embodied carbon dioxide (ECO2)

|  | Material consumption |  | kg embodied CO2 per kg of material | embodied CO2 in kg / m² |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | unreinforced | HaTelit® reinforced |
| Asphalt (~25 kg/cm) | 100 | kg / m² | 0.076 | 7.60 | 7.60 |
| Bituminous emulsion (70%, 0.3 kg/m²) | 0.21 | kg / m² | 0.55 | 0.12 | - |
| Bituminous emulsion (70%, 1.0 kg/m²) | 0.70 | kg / m² | 0.55 | - | 0.39 |
| HaTelit C 40/17 eco asphalt reinforcement | 0.27 | kg / m² | 3.88 | - | 1.05 |
| Total embodied CO2 for rehabilitation | kg / m² |  |  | 7.72 | 9.04 |
| Improvement factor | [ - ] |  |  | 1 | 3 |
| Design life (improved) | years |  |  | 4 | 12 |
| Total embodied CO2 per year design life | kg / m² / year |  |  | 1.93 | 0.75 |
| CO2 saving per m² and year of design life |  |  |  |  | 61 % |
| Total CO2 saving for improved design life |  |  |  |  | 70,800 kg |

The result is a saving of 61 % of ECO2 per m² and year of design life for the HaTelit reinforced overlay as compared to the unreinforced overlay. For a project of 5,000 m² to be repaved this would mean a total ECO2 saving of 70,800 kg based on the significantly improved design life of 12 years.

## Environmental Product Declaration (EPD)

Many conscientious manufacturers provide EPDs for their products and thus provide a basis for project owners to assess all potential environmental impacts of a product, system or solution during its entire life cycle (cradle to grave). Such EPDs are independently assessed and verified as per ISO 14025 and EN 15804. On this basis individual products, systems or construction methods can be compared for their environmental impact during initial construction and over the full service life of the infrastructure asset as shown above.

## Recyclability of HaTelit Asphalt reinforcement

Many researches has demonstrated that the use of asphalt reinforcement show great benefits in road rehabilitation. Until now there has not been a real design method to predict the loads until the fatigue Milling trials were carried out by Huesker in conjunction with “Mischwerk Schwelm” (in 2004) and RWTH Aachen (in 2008) to demonstrate that a polyester grid (HaTelit) can be milled as normal and that the millings can be recycled.

## Investigation of the milling characteristics

A trial length was laid in May 2004 on land at the asphalt mixing plant at Mischwerk Schwelm in Schwelm in Germany to determine the milling characteristics and recyclability of HaTelit-reinforced roads. The construction of the trial length was as follows: A 0.6 kg/m² coating of bitumen emulsion (U70K) was sprayed on to an existing asphalt base and a layer of high-modulus polyester reinforcement installed. The reinforcement was then overlaid with a 40mm thick asphalt surface course. The trial length was removed after about 6 weeks. The milling depth was 50mm; to ensure that in milled below the asphalt reinforcement layer. The milling was carried out using a small milling machine (Wirtgen W 500) with a drum width of 0.50 m.

During the removal process, it was observed that the reinforcement grid had no detrimental effect on the speed accuracy of the milling operation. Likewise, it was found that only short lengths of fibre residue were present in the millings.

Figure 12: Residual fibres in the millings; few fibres remained on the milling drum

As part of tests on the asphalt, the effect of asphalt reinforcement fibres in the asphalt material on its recyclability was investigated. Marshall asphalt test specimens were made from the asphalt binder layer material with and without asphalt reinforcement fibres and their Marshall stability and flow value was determined.

The reference sample was equivalent to the asphalt binder course laid on the test bed. The variant with asphalt reinforcement fibres was made up to have the same aggregate grading and binder content as the reference sample. This was achieved by the controlled addition of the appropriate quantity of uncontaminated aggregate and binder to the recovered asphalt. The aggregate and binder used were from the same batches of aggregate and binder as were used for making the asphalt binder mix. The Asphalt reinforcement fibre content was the major difference between the two variants and the purpose of the tests was to determine the effect of these fibres. There were only relatively small differences with respect to bulk density and void content between the Marshall test specimens used for the tests. The values for Marshall stability and flow were virtually identical. The results for the Marshall test parameters are shown in Table 8.

Table 8: Marshall test results

| Marshall Stability |  |
| --- | --- |
| Reference sample | 8.4 kN |
| With Reinforcement fibres | 8.5 kN |
| Marshall Flow |  |
| Reference sample | 3.6mm |
| With Reinforcement fibres | 4.3mm |

# CONCLUSION

No unified design approach exists for the inclusion of asphalt reinforcement in pavements, however, many researchers have demonstrated that the use of asphalt reinforcement show benefits in road rehabilitation in extending pavement service life, this paper highlights a number of factors that are important for their success. This increase in service life has a number of financial and social economic benefits. The increased uptake of these asphalt reinforcement products, in particular those produced from recycled polyester, will help support the move towards a more circular economy and achieve net zero carbon emissions in the future.

# REFERNECES

BS EN 10319:2015, “Geosynthetics – Wide-width Tensile Test”, International Organization for Standardization

De Bondt, A.H., (1999) “Anti-Reflective Cracking Design of (Reinforced) Asphaltic Overlays”, Ph.D.-thesis, Delft, Netherlands,

Eidgenössische Materialprüfungsanstalt (EMPA), (2017). “Einsatz von Asphaltbewehrungen im Erhaltungsmanagement von Trag- und Deckschichten, Application of Asphalt interlayers for road pavements”.

Elsing, A., Horgan, G., (2019) “Determining key influence factors of a durable asphalt reinforcement”, 18th Annual International Conference on Highways, Airports Pavement Engineering, Infrastructures & Asphalt Technology, Liverpool

Hammond, G.; Jones, C. (2011), "ICE Inventory of Carbon and Energy V 2.0", Sustainable Energy Research Team (SERT), Department of Mechanical Engineering, University of Bath, UK

Höptner, Aaron;  (2010) “Nachweis der Wirksamkeit von Asphaltbewehrungsgittern zur Verhinderung von Reflexionsrissen“, Diplom Thesis, Dresden, 2010

Huang, Yang H.; (1993) “Pavement Analysis and Design” New Jersey: Prentice Hall RWTH Aachen University, (2018). CERTIFICATE No.: 1808991, “Testing of the layer bond of drill cores with and without reinforcement”

Treloar G.J. (1994), "Embodied Energy Analysis of the Construction of Office Buildings", Master of Architecture Thesis, Deakin University, Geelong, Australia Sustainability. (2011, August 16). In Wikipedia, The Free Encyclopedia. Retrieved 05:43, August 17, 2011, from http://en.wikipedia.org/w/index.php?title=Sustainability&oldid=445226512

United Nations General Assembly (1987). "Report of the World Commission on Environment and Development: Our Common Future", Transmitted to the General Assembly as an Annex to document A/42/427 - Development and International Co-operation: Environment; Our Common Future.

WRAP Waste and Resource Action Plan (2010), "Sustainable Geosystems in Civil Engineering Applications", UK

Quiel, M. (2013) „Einsatz von Asphalteinlagen im Asphaltstrassenbau.“ Master Thesis, Münster University of Applied Sciences, Münster.

Elsing, A., Schröer, S., (2005) “Experience from more than 30 years of asphalt reinforcement with polyester grids”, 15th International Road federation World Meeting, Bangkok

The Use of Repetitive Strain Oscillation to Measure Relaxation Capacity in Bitumen

F. Walmsley; R. Haigh; and G. Schofield

Total Energies UK Limited, Bitumen Division, Preston, UK

doi: 10.1515/ijpeat-2016-0049

ABSTRACT

This research makes use of an oscillation time sweep on a Dynamic Shear Rheometer (DSR) (BS EN 14770:2012) to develop a novel 30-minute Repeated Oscillation Test to assess how different bituminous materials at different stages of ageing respond to dynamic repeated strain. This provides the ability to measure and quantify how various bitumen samples respond to induced strain, which can then potentially be used to relate how a binder might respond to fatigue. This work expands on previous binder fatigue work by using a range of different shear strain levels to produce results which are representative of the different types of road loading experienced by bitumen in situ. A variety of virgin and aged bitumen samples were employed for this research, ranging from penetration grade to highly polymer modified bitumen (PMB) to provide a comparative set of results. Change of Complex Modulus (G*) is used to calculate how the bitumen samples reacted to the shear strain.

1	INTRODUCTION

Bitumen fatigue has always been an area that the asphalt and bitumen industry has struggled to quantify, characterise, and accurately test. This is because fatigue is not just one measurable aspect, instead it is made up of multiple aspects that make up and cause fatigue. While tests such as the Texas Overlay Test (TOT) and 4-Point Bending (BS EN 12697-26:2018) have been developed using stress and strain to produce results relative to the fatigue resistance of a bituminous mixture, there are few to no reliable testing procedures that can show the fatigue resistance of a bitumen binder. Fatigue testing is of great importance; not only can fatigue resistance tests show how the bituminous mixture will perform in situ under repeated loading from vehicles, but it will also be directly related to the lifespan of the bituminous mixture in-situ before it begins to crack and fail. If the fatigue resistance of a binder or bituminous mixture can be calculated, these results can then be considered in the design and more specifically, deterioration modelling to enhance asset management design and prediction of preventative maintenance. This in turn should enhance the durability of the road and reduce overall lifetime costs in terms of both money and carbon.

Bitumen binder fatigue has, and continues to be, researched by many in the industry, who are trying to find ways to characterise the phenomenon and be able to produce reliable test results. Many of these researchers have gone down the route of using rheological testing such as the Dynamic Shear Rheometer (DSR) to produce fatigue characteristic and results (Johnson et al, 2009; Shenoy 2002; Tapsoba et al, 2013). These rheological tests tend to either put an axial strain or stress on to a bitumen sample, which in turn produces results that usually show a decrease of the Complex Modulus (G*), relating to the loss of stiffness of the sample (Tapsoba et al,2013). One of the issues that comes with using DSR to characterise fatigue in a bitumen binder is due to the very nature of bitumen. Bitumen is a thixotropic fluid, meaning that its viscosity reduces when stress is applied. This thixotropy makes its very difficult to define the line between what is a fatigue characteristic and what is a thixotropic characteristic of the bitumen; when either stress or strain is applied to the bitumen it causes a sudden decrease in viscosity and therefore a reduction in the stiffness of the bitumen. Consequently, this is shown in the results via an exponential decrease in G*.

The author recognises that true fatigue is a complexed multi-dimensional characteristic that is not simple to define. Therefore, the work presented in this paper uses the research of both past and present fatigue works and consolidates this research, to produce a repeated oscillation test using multiple strains to investigate the response characteristics of the different bitumen types and how ageing affects that response.

The research by Aroon Shenoy 2002 (Shenoy, 2002) focused the research on the fatigue testing of penetration grade binders, this was done by using the Dynamic Shear Rheometer (DSR) to produce results from an extended oscillation time sweep at a 25% shear strain. All the samples used in the research were tested at 25°C and at a frequency of 10rads/s. Initially the research ran the oscillation time sweep over 16 hours, which was around 576,000 cycles, as the research progressed a proper window of fatigue failure was introduced at 50% fatigue point of failure; using the loss of Complex Modulus (G*) to define this fatigue failure point. This significantly reduced the time of the oscillation time sweep down and the average failure point of the penetration bitumen was between 250-750 cycles.

The research in this paper used the research produced by Aroon Shenoy 2002 (Shenoy, 2002) to form the foundation of this test; therefore, the test makes use of a Dynamic Shear Rheometer (DSR) to produce results from an extended oscillation time sweep at multiple strain levels. Unlike previous research this regime uses both virgin and aged bitumen samples, across different penetration grade and polymer modified bitumens (PMBs).

The change in complex shear modulus (G*) through the test is used to characterise the induced response within the bitumen samples used in this research. G* is employed for this role as it is inherently related to the stiffness, particularly through the contribution of the elastic modulus (G’); therefore, the higher the G* the stiffer the material. This makes G* the perfect component to use to characterise the induced response within the bitumen, as progressive ageing increases the G’ contribution and reduces the G’’ contribution making the bitumen less able to relax induced strain consequently making it vulnerable to micro-cracking. This can be shown in the test as the aged samples are stiffer and therefore demonstrate weaker resistance to the induced strain which is shown via higher percentage losses in G*.

This testing regime is comparable with the TOT regime because both sets of testing methodology make use of repetitive strains to create stress within samples. However, to maintain comparability with 4-Point Bending as well, it was decided that a sample had reached a point of failure when it’s loss in G* reached 50%. It was found through testing that multiple strain levels would provide more information about how the bitumen behaves. For instance, PMBs and any binder intended for heavy loading roads and/or roads with an unstable or mobile subsurface; where physical or thermal strain can be an issue, should be tested using a higher strain level. Whereas unmodified binders and any binder intended for lower loading roads should use a lower strain. This led to using a variety of different strain levels ranging from 5-25% depending upon the grade of binder and intended use of the binder in situ.

2	MATERIALS AND METHODOLOGY

2.1	Materials

This range of testing used two sets of binders. Each set comprised of four binders which were a mixture of both PMB and penetration grade binders. Set one contained the virgin binders Styrelf eXtreme 100(Premium PMB), Styrelf eXtreme 100 Long Life (Premium PMB with Low ageing properties), Styrelf eXcel 70 (intermediate grade PMB) and 40/60. The second set contained the same binder grades with the difference that these binders were subject to long term ageing for 25, 48 and 72 hours in a Pressurised Ageing Vessel (PAV) (BS EN 14769:2012), to simulate the binders after they have been in situ for a prolonged period.

| Name of Binder | Type of Binder |
| --- | --- |
| Styrelf eXtreme 100 | PMB 75/130-75 |
| Styrelf eXtreme 100 Long Life | PMB 75/130-75 With Reduced Ageing Technology |
| Styrelf eXcel 70 | PMB 45/80-50 |
| 40/60 | 40/60 Penetration Grade |

Table 1 – Types of binder employed throughout the testing.

2.2	Equipment

An oscillation time sweep on a Malvern Kinexus Lab+ Dynamic Shear Rheometer was used with a set of 8 mm parallel plates to conduct the repeated strain testing. This allowed for use of a set temperature, frequency, and oscillation, while investigating different shear strains. All the samples were created using an 8mm silicone rubber mould.

All aged samples were aged in a Pressurised Aging Vessel at set times 25, 48, 72 Hours, pressure of 2.1MPa and temperature 100°C.

2.3	Establishing Proper Experimental Procedure

Before a full procedure could be created, several experimental runs were completed on each binder using an oscillation time sweep with different conditions. This enabled the author to establish optimum testing conditions that would produce reliable and comparable results for all the binders. Several different experimental conditions were trialled during the initial stages of this research, such as different sizes of parallel plates used. The 8mm parallel plates were found to be more sensitive and gave more reproducible results to the 25mm parallel plates; due to the 25mm plate producing too much noise during testing therefore, the 8mm plates were used for the rest of the research. Different loading and testing temperatures were also trialled during this stage but ultimately it was decided that it would be best to stick with the temperatures from Aroon Shenoy’s 2002 research (Shenoy, 2002). Therefore, it was agreed to continue research using a 45°C loading temperature and a 25°C testing temperature because they produced the best repeatability of results when samples were loaded and tested at these temperatures. The loading temperature chosen is standard with other rheology testing using similar equipment and the testing temperature chosen is the standard temperature used for testing done on bitumen and asphalt such as, Binder Penetration (BS EN 1426:2015)

From the initial testing stage of the research, it was found that most of the testing conditions such as plate size, loading temperature, testing temperature and oscillation frequency could be kept constant throughout the testing on all the different binders. One noticeable thing that the experimental runs did reveal was that the strain levels could be adjusted between 5-25% shear strain depending on the grade and the intended use of the of binder in situ. The results produced between 5-25% shear strain produced the most reliable range of results and means that different grades can be tested on a strain that is within the linear viscoelastic (LVE) region of testing for each binder. It was found that PMB grades are within the LVE region at a 25% shear strain, however penetration grades instantly fail at 25%, therefore a range of 5-15% shear stain needed to be used for a penetration grade bitumen.

2.4	Methodology

Each of the binder samples were manufactured and split into smaller sample sizes to either be retained as virgin binder samples or to be aged in the Pressurised Ageing Vessel (PAV) for either 25, 48 or 72 hours at 100°C. The 25, 48 and 72 hours ageing times were chosen so that the aged binders were comparable with previous in house testing done at these ageing times and conditions. While waiting for testing, all the virgin and aged binder samples were stored in a 5°C cold store to prevent any further ageing of the samples (BS EN 12584:2014).

Before the Repeated Oscillation test was run, each binder used was heated following the BS EN 12594 Standard and then poured into an 8mm silicone rubber mould, where it was then allowed to cool for 30 minutes. While the sample was cooling the 8mm parallel plates would be placed into the DSR and the Repeated Oscillation Test program would be loaded. Following the program, the 8mm parallel plates were heated to a loading temperature of 45°C, to allow the binder samples to fully adhere to the plates once loaded. It took 15 minutes for the plates to reach the correct 45°C loading temperature and to stabilise.

Once the binder samples had cooled, they were removed from the silicone mould and loaded onto the underside of the top 8mm parallel plate. After the sample was loaded on to the top plate, the plates were lowered, and the sample was compressed between the two 8mm parallel plates until the sample reached the desired thickness of 2mm. Any excess sample was trimmed off from around the plates using a hot spatula.

The temperature of the plates and sample were then lowered to the testing temperature of 25°C. Each sample was left for 15 minutes to allow the new temperature of 25°C to be reached and stabilise before testing began. A Dynamic Shear Rheometer (DSR) was used to run the Repeated Oscillation test. The DSR used a program which was set up to run each test for 30 minutes, while collecting and recording data every 5 seconds throughout the entire duration of the test. The first data reading is taken after 10 seconds after the test has started because it takes 10 seconds for the shear strain to get up to the correct percentage. The tests were run at a testing temperature of 25°C, on 8mm parallel plates that moved at a frequency of 10 rads/s. The shear strain was the only parameter varied from test to test and was selected by the operator dependent upon how much stress needed to be induced to the binder. The operator would choose a shear strain from a range between 5-25%. At the end of each test the program would compile all the data into a data table. When each test was complete the sample would be removed, and the machine cleaned. Then all the data would be collected from the DSR program and recorded appropriately.

3	RESULTS

3.1	Virgin Binder Test Results

Figure 1 shows the data from four Repeated Oscillation Tests on the 40/60 binder, plotted as the Loss of Complex Modulus (G*) versus Time (seconds). The tests were run at 25°C and at a constant frequency of 10 rad/s. For each of the tests the shear strain was varied so that all four tests were run at different shear strain levels (5, 10, 15 and 25% respectively). A difference was seen between the four tests, in agreement with previous research (Shenoy, 2002). However, it was decided to normalise the Complex Modulus

Normalised G*=Complex modulus G*Initial complex modulus(G")

so that all the data started at the same point and use it to calculate the retained percentage of G*, thereby making it easier to compare the data and visualise the failure point of 50%. Figure 2 shows the same data from Figure 1 plotted as the Retained Percentage of G*(%) versus Time (seconds). Using the Retained Percentage of G* made it easier to compare how the different shear strain levels effect the rate of decline in the samples.

Figure 13 - Plot of the Loss of Complex Modulus (G*) of 40/60 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

It should be noted that the reason the Figure 1 graph is showing a different start point G* figures is due to the 10 second delay of the first reading. The induced strain has already created damage to the sample prior to that first reading, hence the lower initial G* for the higher strain tests.

Figure 14 - Plot of the Retained Percentage of G* (%) of 40/60 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

Figure 15  - Plot of the Loss of Complex Modulus (G*) of Styrelf eXcel 70 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

Figure 3 shows the data from four Repeated Oscillation Tests on Styrelf eXcel 70 binder, plotted as the Loss of G* versus Time (seconds). The tests were run at 25 °C and at a constant frequency of 10 rad/s. For each of the tests the shear strain percentage was varied so that all four tests were run at different shear strain levels (5, 10, 15 and 25% respectively). Figure 4 shows the same data from Figure 3 plotted as the Retained Percentage of G*(%) versus Time (seconds).

Figure 16 - Plot of the Retained Percentage of G*(%) of Styrelf eXcel 70 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

Figure 5 shows the data from 4 Repeated Oscillation Tests on Styrelf eXtreme 100 binder, plotted as the Loss of G* versus Time (seconds). The tests were run at 25°C and at a constant frequency of 10 rad/s. For each of the tests the shear strain percentage was changed so that all four tests were run at different shear strain levels (5, 10, 15 and 25% respectively). Figure 6 shows the same data from Figure 5 plotted as the Retained Percentage of G*(%) versus Time (seconds).

Figure 17 - Plot of the Loss of Complex Modulus (G*) of Styrelf eXtreme 100 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

Figure 18 - Plot of the Retained Percentage of G*(%) of Styrelf eXtreme 100 binder over 30 minutes of the Repeated Oscillation Test at various shear strain percentages.

3.2	Aged Binder Test Results

Figure 7 shows the data from eight Repeated Oscillation Tests on three different binders: Styrelf eXtreme 100, Styrelf eXtreme 100 Long Life and Styrelf eXcel 70. Three of the tests were carried out on the virgin binders, while the remaining five tests were run on the same samples after ageing using a Pressurised Ageing Vessel (PAV).  The data from these tests is plotted in Figure 7 as the Loss of G* versus Time (seconds). The tests were run at 25°C and at a constant frequency of 10 rad/s. For each of the tests the shear strain percentage was kept constant at 25%. Figure 8 shows the same data from Figure 7 plotted as the Retained Percentage of G*(%) versus Time (seconds).

Figure 19 - Plot of the Loss of Complex Modulus (G*) over 30 minutes of the Repeated Oscillation Test at 25% shear strain.

Figure 8 - Plot of the Retained Percentage of G*(%) over 30 minutes of the Repeated Oscillation Test at 25% shear strain.

Figure 9 - Bar chart of the Loss of Complex Modulus (G*) and Percentage loss of the Complex Modulus over 30 minutes of the Repeated Oscillation Test at 25% shear strain.

Figure 9 shows the same data from Figure 7 plotted as a bar chart, with the total loss of the Complex Modulus on one axis and the percentage loss of the G* on the other axis. Presenting the results this way allowed for a better overall comparison of the materials, comparing their total decline rather than showing their decline throughout the test.

4	DISCUSSION

The results from the Repeated Oscillation test have shown that stress can be induced into a binder using repeated oscillations at multiple strains. The capacity of the binder to relax the induced strain is well illustrated by the degree of loss of G* through the test. The results have clearly shown the different behaviours of various types of binder, specifically related to relaxation capacity and potentially indirectly the relative fatigue characteristics of binders. The relaxation capacity characteristics seen in the results show significant differences between penetration binder and PMBs, as well as considerable differences between virgin binder and aged binders.

Figure 1 and Figure 2 show the results from the 40/60 penetration grade at four different shear strain levels. Figure 1 shows that using different shear strain levels resulted in different initial G* values of the 40/60. For instance, at 25% shear strain the initial G* value of the 40/60 is lower, whereas at 5% shear strain the initial G* value is significantly higher. However, while the different shear strains show a difference in the initial G* reading it is likely that this initial G* difference is down to the amount of damage done in the first 10 seconds of the test to the sample. This trend seen in the 40/60 testing range was also seen throughout testing with the rest of the binders, both virgin and aged. Due to the differences in the initial G* value for each test, the results were converted into Normalised G* and then used to calculate the Retained Percentage of G* so that every result had the same starting point.

Figure 2 shows the 40/60 results presented as the Retained Percentage of G*; by presenting the results in this way it makes it easier to compare the different strains used and to see the failure point (50% loss of G*) of the binders.  Figure 2 shows a considerable difference between results when the shear strain level was changed. At 25% shear strain the 40/60 binder failed within the first 100 seconds; similarly, at 15% shear strain the 40/60 binder failed within the first 300 seconds. Both the 15% and 25% shear strain failed within the first 5 minutes of the test, meaning these strains are too high and cause the samples to rapidly decline from the start of the test, leading to failure very quickly. These shear strain levels were repeated multiple times and the same result was achieved each time. This could also be interpreted as showing that the 40/60 binder is unsuitable to be used as the binder on high loading and roads more susceptible to movement through thermal actions or instability in the lower layers; as it is more susceptible to the induced stress on the higher levels of shear strain and therefore would also likely show high levels of fatigue in a relatively short time period and fail on higher loading roads very quickly.

Figure 2 also shows the test on the 40/60 binder run at 10% and 5% shear strain. In these tests, the 40/60 binder endured to 750 seconds before failure on the 10% shear strain, while on the 5% shear strain it did not reach the failure point of 50% loss. This implies that the 5% shear strain is too low and does not have enough impact on the sample to cause failure, which was also seen in other samples tested at 5%. However, the 5% and 10% could be compared to lower loading roads and therefore show that the 40/60 binder would last longer in situ on the lower loading roads than the higher loading roads. This is shown by the slower decline in the results which therefore demonstrates the retained capacity to relax the induced strain and the relative fatigue rate of the binder at these levels is likely to be lower and thus any asphalt containing this binder would last longer on lower loading roads as opposed to higher loading roads.

Figure 3 and Figure 4 show the results from the Styrelf eXcel 70 binder at four different shear strain levels. Figure 3 shows that using different shear strain levels resulted in different initial G* values of the eXcel 70, which was also observed for the 40/60. Just like with other binders, at 25% shear strain the initial G* value of the Styrelf eXcel 70 is lower, whereas at 5% shear strain the initial G* value is significantly higher. Figure 4 shows the Styrelf eXcel 70 results from Figure 3 presented as the Retained Percentage of G *, and these results show differences when the shear strain level was changed. At 25% shear strain the Styrelf eXcel 70 binder failed within the first 200 seconds, while at 15% shear strain the Styrelf eXcel 70 binder failed within the first 750 seconds. So, whilst the Styrelf eXcel 70 illustrates a significantly improved capacity to relaxed low and mid-levels of induced strain over the 40/60 binder, it struggles to maintain the capacity to relax the higher levels of strain. At 25% shear strain the binder failed within the first 6 minutes of the test, suggesting that this strain is too high, causing the sample to rapidly decline from the start of the test and fail very quickly. The shear strain levels were repeated multiple times and the same result was achieved each time. This implies that, the Styrelf eXcel 70 binder is not ideally suited to be used in situations where high levels of strain are likely to be encountered, for example where it is likely to be exposed to thermal expansions and contractions associated with cementitious overlays. However, at 15% it did take 750 seconds for the Styrelf eXcel 70 to fail, meaning that Styrelf eXcel 70 could be used on a moderate strain loading situation.

Figure 4 also shows that the Styrelf eXcel 70 binder did not reach the failure point of 50% loss on either the 5% or 10% shear strain parameters. For this reason, it is reasonable to say that the 5% and 10% shear strain are too low for this binder and do not have enough impact on the sample to cause failure. The 5% and 10% could be compared to lower loading roads and show that the Styrelf eXcel 70 would take longer to fatigue and therefore take longer to fail in situ on the lower loading roads. This is shown by the slower decline in the results illustrating the susceptibility to the induced stress of the binder at these levels is lower than the 40/60 bitumen and therefore any asphalt containing this binder would last longer. Looking at the results from the Repeated Oscillation test it proves that Styrelf eXcel 70 would perform best on moderate to low strain loading roads.

Figures 5 and 6 show the results from the Styrelf eXtreme 100 binder at three different shear strain levels. Figure 5 shows that just like with the 40/60 and eXcel 70 testing, using different shear strain levels resulted in different initial G* values of the Styrelf eXtreme 100. Also, in agreement with the other binders, at 25% shear strain the initial G* value of the eXtreme 100 is lower, whereas at 5% shear strain the initial G* value is significantly higher. This difference being the resultant damage caused during the first 10 seconds of loading. Figure 6 shows the Styrelf eXtreme 100 results from Figure 5 presented as Retained Percentage of G*, demonstrating differences between results when the shear strain level was changed. At all three shear strain levels the eXtreme 100 did not reach the 50% loss failure point. Even though the Styrelf eXtreme 100 did not fail at any shear strain, it still followed the pattern of the other binders whereby the higher the shear strain the more stress the sample experienced and the less capacity the binder had to relax the strain. 25% shear strain showed the highest loss of G* and had the lowest retained percentage of G* under 55%. At 15% shear strain G* had a retained percentage of under 75% whereas 5% shear strain had a retained percentage of over 95%. The results from the Repeated Oscillation test on the Styrelf eXtreme 100 show that the binder does not fail at any of the shear strain levels and would therefore be suited for any type of road and could withstand low to high strain loading roads. This binder would also withstand roads with higher levels of stress in the lower layers caused by underlying structural issues, making it ideal for use in overlaying jointed concrete and the like; this makes Styrelf eXtreme 100 ideal for stress absorbing interlayers.

Comparing Figure 4 and Figure 6 the results show that the Repeated Oscillation Test produces results showcasing how different levels of polymer in PMBs react during the test. Styrelf eXcel 70 has a much lower polymer content than Styrelf eXtreme 100 and this is seen in Figure 4 and Figure 6, where the Styrelf eXcel 70 results (Figure 4) show a faster decline and failure during in testing; indicating a lesser capacity to relax the higher levels of induced strain. In comparison, the Styrelf eXtreme 100 (Figure 6) results show that the increased polymer content in the Styrelf eXtreme 100 can withstand the induced stress better than Styrelf eXcel 70, and ultimately because of the higher polymer content the eXtreme 100 never failed on any of the different shear strain levels.

Figure 10 - Plot of the Retained Percentage of G*(%) over 30 minutes of the Repeated Oscillation Test at 25%, 15% and 5% shear strain.

Figure 10 shows the combined results of Styrelf eXtreme 100 and the 40/60 penetration grade at three different shear strain percentages (5%, 15% and 25% respectively) presented in a normalised complex modulus graph. The Repeated Oscillation test results show a clear difference in the characteristic of relaxation capacity between a penetration grade and a PMB grade. All the samples at each shear strain show an initial decline in G*, however a difference is truly seen between the two types of binders by the continued decline in G* in the penetration grade even with the 5% shear strain sample. Whereas, in the PMB grade there is the initial drop in G* which then levels out and is maintained throughout the rest of the test. This suggests that the test does not span over a long enough length of time to induce the stress in the PMB that is needed to cause it to reach the 50% loss of G* to fail. The penetration grade in each of the shear strains shows a clear decline in G*, and even the 5% shear strain sample which did not reach failure point still clearly shows the start of decline of G* and would presumably reach the failure point if the test was continued past 30 minutes.

The comparison of the different bitumen grades is also show in Figure 11, which displays the results from Figure 10 presented as a bar chart presenting the total loss of G* and the percentage loss of G*. The comparison in Figure 11 also helps to highlight the different characteristics of the two types of bitumen, showing that the induced stress of the Repeated Oscillation test affects the penetration grade more significantly than the PMB. This is seen as the penetration grade samples have both a higher total loss of G* and a higher percentage loss of G* over all three different shear strains compared to the PMB results. This indicates how differently the two type of bitumen’s perform in situ. For instance, the results would suggest that the penetration grade fatigues, loses its relaxation capacity and fails quicker than the PMB regardless of loading. However, the penetration grade still shows that it would perform to a satisfactory standard in a low strain loading road. Whereas the PMB grade would work on any road regardless of whether the road is low or high loading. However, due to the PMB results showing lower susceptibility to fatigue, the PMB is best suited to moderate to high loading road depending on the polymer content, so that it can reach its full potential and provide best value.

Figure 11 - Bar chart of the Loss of Complex Modulus (G*) and Percentage loss of the Complex Modulus over 30 minutes of the Repeated Oscillation Test at 25% shear strain.

The success of the Repeated Oscillation test on virgin binder with induced stress showed clear and significant differences between characteristic of the different types of binders. Consequently, the decision was made to age the virgin binders by 25, 48 and 72 hours in a PAV (Pressurised Ageing Vessel) to see if the Repeated Oscillation test would show any changes in characteristics of the binders before and after ageing. For this, eXtreme 100 Long Life was also used as it is specifically manufactured to slow down the oxidation process, therefore reducing the binders aging. This binder was tested as both virgin and as an aged sample against the standard eXtreme 100 binder to see if the Repeated Oscillation test could pick up any difference in characteristics between the two samples. Both Figures 7 and 8 show the results for the virgin and aged samples of three binders run at 25% shear strain; Styrelf eXcel 70, Styrelf eXtreme 100 and Styrelf eXtreme 100 Long Life. Figure 7 shows the results for the samples as the Loss of G* over the course of the test run, and Figure 8 shows the results presented as normalised G*.

The results from the Repeated Oscillation testing on the aged binder samples (Figure 7 and Figure 8) show a clear and significant difference in characteristics between the aged binders. The aged samples have higher initial G* values compared to the virgin binders, reflecting the oxidation that has taken place. The aged samples also show a stronger reaction (reduced capacity to relax strain) to the induced stress of the Repeated Oscillation test, which can be seen in Figure 8 as they show a more rapid decline in the Retained Percentage of G* compared to the virgin binders.  Figure 9 presents the results from Figure 8 as a bar chart showing the total loss of G* and the total percentage loss of G*. These results clearly show how different lengths of ageing influence the characteristics of a sample when strain is induced. Figure 9 shows results for four eXtreme 100 samples all aged for different lengths of time: virgin, 25 hours aged, 48 hours aged, 72 hours aged. A consistent increase in the total loss of G* and the total percentage loss of G* is observed, showing that the Repeated Oscillation test can characterise different aged samples when subjected to induced stress. This shows that the test has potential uses in estimating how long a binder will last in situ before it fails and must be replaced. Consequently, this could also be used to predict when it is likely for a road containing the binder to need preventative work to prolong its lifespan i.e., deterioration modelling.

As shown in Figure 8, the Styrelf eXtreme 100 and Styrelf eXtreme 100 Long Life virgin binders both have the same results. However, the aged samples of these binders show the differences during testing; for instance, the results from the Styrelf eXtreme 100 sample aged for 48 hours produced similar results (capacity to relax strain) to the Styrelf eXtreme 100 Long life sample aged for 72 hours.  This shows that the age reduction process used to manufacture Styrelf eXtreme 100 Long Life does decrease the binders ageing and the Repeated Oscillation test is able to record these characteristics of the product.

5	CONCLUSION

In conclusion, the Repeated Oscillation Test has been proven to show the characteristics of different types of bitumen in response to applied shear strain, both before and after sample ageing. The test was able to show that inducing stress through applied strain in a penetration grade bitumen will cause it to lose the capacity to relax that strain and potentially fatigue and fail more rapidly than a PMB, regardless of the level of shear strain used.  The Repeated Oscillation Test can therefore be used to predict how long, or how well a binder will last on a particular section of road, by using the different levels of shear strain as a substitute for the different levels of loading seen on different roads. More research will need to be conducted to assess the comparability of the shear strain levels against the different levels of road loading.

The Repeated Oscillation Test showed the differences between how both virgin and aged binders react to induced strain, proving that as expected the virgin binder outperformed the aged binders. As the ageing of the binders increased the binders capacity to relax the strain decreased. With further research and testing, the combination of artificial ageing of binders such as RTFOT (BS EN 12607-1:2014) and/or PAV and the Repeated Oscillation Test could be used to predict the life span of a binder and therefore the road is it used in. Not only could the combination of testing predict when the road might fail, but it could potentially be used to help predict when a road might need preventative work, such as resurfacing intervention, to help prolong the roads lifespan. To do this, further research should also include taking core samples from roads and reclaiming the binders to see how it compares to the RTFOT and PAV binder samples during testing in this type of analysis. This further research will also work on quantifying and correlating how the 5-25% shear strain level compared to binder that is in situ in asphalt.

Future research will also include comparisons between the Repeated Oscillation Testing and asphalt fatigue testing, such as the 4 Point Bending Test and the Texas Overlay Test. This future research will help to establish if the Repeated Oscillation Test is simply just measuring the binder’s capacity to relax strain or if it is indeed testing fatigue levels in a binder sample. This comparison of asphalt and binder testing will help to determine if the Repeated Oscillation Test is a true Fatigue test that can be used on binders.

7	REFERENCES

BSI Standards Publication. Bitumen and bituminous binders – Accelerated long-term ageing conditioning by a Pressure Ageing Vessel (PAV). BS EN 14769:2012. Published by BSI Standards Limited 2012.

BSI Standards Publication. Bitumen and bituminous binders – Determination of complex shear modulus and phase angle – Dynamic Shear Rheometer (DSR). BS EN 14770:2012. Published by BSI Standards Limited 2012.

BSI Standards Publication. Bitumen and bituminous binders – Determination of needle penetration. BS EN 1426:2015. Published by BSI Standards Limited 2015.

BSI Standards Publication. Bitumen and bituminous binders – Preparation of samples. BS EN 12594:2014. Published by BSI Standards Limited 2014.

BSI Standards Publication. Bitumen and bituminous binders – Determination of the resistance to hardening under influence of heat and air Part 1: RTFOT method. BS EN 12607-1:2014. Published by BSI Standards Limited 2014.

BSI Standards Publication. Bituminous mixtures – Test Methods Part 26: Stiffness. BS EN 12697:2018. Annex B. Published by BSI Standards Limited 2018.

Johnson, C., Bahia, H. and Coenen, A., 2009. Comparison of Bitumen Fatigue Testing Procedures Measured in Shear and Correlations with Four-Point Bending Mixture Fatigue. 2nd Workshop on Four Point Bending, Pais (ed.).

Shenoy, A., 2002. Fatigue Testing and Evaluation of Asphalt Binders Using the Dynamic Shear Rheometer. Journal of Testing and Evaluation, 30(4), p.303.

Tapsoba, N., Sauzéat, C. and Benedetto, H., 2013. Analysis of Fatigue Test for Bituminous Mixtures. Journal of Materials in Civil Engineering, 25(6), pp.701-710.

Thermal Fatigue Analysis of Pavement Design using Hydrated Lime Modified Asphalt Concrete

Yu Wang 1*, Azedin Al Ashaibi 1, Amjad Albayati 2, Jonathan Haynes 1,

1 School of Science, Engineering & Environment, University of Salford, UK

2 Department of Civil Engineering, University of Baghdad, Iraq

*Corresponding author: y.wang@salford.ac.uk

doi: 10.1515/ijpeat-2016-0050

Abstract

A previous experimental study has suggested that asphalt concrete using hydrated lime to partially replace the conventional limestone dust filler at 2.5% by the total weight of all aggregates had an optimum improvement on the key mechanical properties, fatigue life span and moisture susceptibility. To promote the real-world application, more knowledge of the thermal response of the modified asphalt concrete is needed to provide the required information for pavement design. For the purpose, an experimental test, under three temperature conditions, has been conducted to study the fatigue of an optimum mix for wearing surface. Based on the experimental data, a thermal-fatigue characterization model has been derived for the asphalt mix. At the end the model has been implemented on a case investigation to analyse the seasonal thermal effect on the fatigue of pavement constructed using the optimum hydrated lime modified asphalt concrete.

Keywords: Asphalt pavements, Hydrated lime additive, Thermal stress, Fatigue Analysis

- Introduction

Flexible pavements are subjected to three main distresses: fatigue crack, thermal crack and rutting. The combined effect of repetitive traffic loading and the seasonal temperature variation plays the key role in the activating and development of the distresses, and particularly the premature deterioration of the pavement infrastructures [Souza and Castro 2012]. Modifying asphalt concrete using mineral additives has been widely adopted and in the efforts to improve pavement durability. Among the popularly used mineral additives, hydrated lime [Ca(OH)2] (HL) has been found, for a long time, of particular interest due to both the effectiveness and cheap cost [Lesueur et al. 2013]. A recent experimental study [Al-Tameemi et al. 2016, 2019] has suggested to replace conventional limestone filler using HL by 2.5% of the total weight of aggregates produced an optimum enhancement on the uniaxial permanent deformation, residual modulus, moisture susceptibility and fatigue life of the asphalt mixes for three different layer applications, i.e., wearing, levelling and base.

Directly exposed to atmosphere, pavement surfaces experience a combined deteriorating mechanism caused by traffic loading, environmental temperature variation and solar radiation. However, so far, little experimental study has been found for the thermal effect on the fatigue life of the asphalt concrete modified using HL [Bourona et al. 2021, Albayati and Mahir, 2015]. Meanwhile few research works have reported a detailed computational analysis for the climatic impact on the fatigue life of the pavement structures constructed using the HL modified asphalt concrete [Kai and Fang 2011, Xu and Solaimanian 2010]. To increase the knowledge in these aspects, this paper reports a laboratory experimental study of the fatigue life of a hydrated lime modified mix for wearing course application under three different temperature conditions. Based on the experimental data, a thermal-fatigue characterization model has been derived for the specific asphalt mix. At the end, the model was implemented to analyse the combined effect of traffic loading and seasonal temperature variation on the fatigue life of a pavement structure constructed using the optimally hydrated lime modified asphalt concrete.

- Experiment of Thermal Effect on Fatigue Life

An experiment was performed to study the temperature effect on six different mixes designed for the wearing course application according to the Standard for Road and Bridge Construction [SCRB/R9 2003]. A control mix was that used limestone dust for the mineral filler by 7% of the total weight of aggregates. The five other mixes used HL to replace the limestone dust in the control mix at a rate of 1, 1.5, 2, 2.5 and 3%, respectively, by the total weight of aggregate. All mixes were casted into specimens in a form of beam of a dimension of 76 (W) × 76 (D) × 381 (L) mm. These beam specimens were tested for fatigue life using the four-point bending method under three temperatures of 15 ̊C, 20 ̊C and 25 ̊C, respectively. Four vertical downward loads were applied for each mix, they were 223, 310, 402, and 490 N, respectively. The loads were applied in a 2 Hz cyclic rectangular form with 0.1 second of loading followed by 0.4 seconds of resting (no loading) [Huang 2004]. In each test, an initial tensile strain at the bottom of middle span was recorded at the 200th repetition. Further detailed information about the mix design, sample preparation and fatigue test, etc. can be found in a previous work [Al-Tameemi et al. 2019].

Figure 1 shows the fatigue test results, the S-N curves. It can be seen from Fig. 1(b) that the trend lines of mix of 2.5% HL under the three temperatures have showed of the lowest slope. Meanwhile, given the same initial strain, the 2.5% HL mix has the best or close to the best fatigue life – of the highest load repetition number to failure, under temperature 15 ̊C and 25 ̊C, and given a low initial stain (less than 270 micro-strain) under 20 ̊C. The result confirms and strengthens again the conclusion in previous study that 2.5% HL addition is the optimum for HMA concrete to obtain a comprehensive improvement on material properties when exposed to both mechanical loading and thermal influences.

- in normal scale

- in log-log scale

Fig. 1. Fatigue test results of the 6 mixes for wearing course application

Figure 2 highlights a comparison of the results for the mix of 2.5% HL in the Fig. 1.

Fig. 2. The S-N curve of the mix of 2.5% HL under three temperatures and the fitting at log-log scale

A power function, εt_200th=k1Nfk2, is used to represent the S-N curves under three temperatures. The fitting was conducted in the log-log scale, which presents in a linear form. Fig. 3 show the variation of the parameters k1 and k2 with the temperature. At last, we can characterise the thermal effect on the S-N curve of the wearing mixes of 2.5% HL addition as:

εt_200th=-1.104×10-4T+7.835×10-3Nf-4.5×10-3T-0.2	(1)

where εt_200th is the strain recorded at 200th repetition, T is the temperature in  ̊C, Nf is the fatigue life - the maximum repetition number of a certain strain before failure.

Fig. 3. Characterization of the k1 and k2 variation with temperature

- Modelling the Coupled Thermomechanical Effect on Pavement Structures

  - Governing equations

A classical thermomechanical model [Little et al. 2018] is adopted to describe the coupled thermomechanical effect on pavement structure and to compare the predicted fatigue life for the pavements using the asphalt concrete of 0% and 2.5% HL addition, respectively.

- Motion equation

For an isotropic solid material, its deformation follows the Navier's equations, which can be written in the form below:

E21+v11-2v∇∇∙u+∇2u+f=ρ∂2u∂t2				(2),

where u stands for the vector of displacement in space (ux uy uz), f is a body force per unit volume, E is modulus and v is Poisson ratio, ρ is density. The strain and stress of the solid material are:

εij=12∂ui∂xj+∂uj∂xi							(3),

σij=λεkkδij+2μεij-α3λ+2μ∆Tδij				(4),

where λ=vE(1-2v)(1-2v) and μ=E2(1+v) are the Lamé parameters, α is the thermal expansion/contraction coefficient, ΔT is the temperature change. The deformation effect on material density can been considered by the Eq. (5) below. However, this effect was neglected in this study.

ρ=1+εxx+εyy+εxxεyy						(5)

- Energy equation

The thermal energy conservation of solid materials is:

ρCpdTdt=∇(k∇T)+ρr-α(3λ+2μ)Tdεkkdt				(6),

where Cp is speciﬁc heat, k is thermal conductivity, r is heat source per unit mass. For current problem the last term of the thermal deformation rate energy is neglectable.

  - Material properties

Solving the equation system of the Eqs (2)-(6) needs the materials’ physical, thermal and mechanical properties, and their variation characterization under different temperatures. Table 1 gives out these relevant properties for the asphalt concrete mixes using 2.5% HL and no HL, which have been investigated before in line with the fatigue experiment study been reported above [Al-Tameemi et al. 2016; Al-Ashaibi et al. 2020]. A thermal deformation model [Islam & Tarefder 2015] based on a laboratory study on a new constructed asphalt pavement, which uses two quadratic polynomials in the form of the Eq. (7) to estimate the coefficients of thermal expansion (CTE) and contraction (CTC) for their variation with temperature, was adopted for this study.

CTC= -1.6×10-8T2+9.5×10-7T+1.7×10-5		(7a)

CTE= 4.3×10-9T2-2.3×10-7T+2.8×10-5		(7b)

The data for the two foundation layers (subbase and subgrade) are referred to another published work [Ishikawa & Miura, 2015].

Table 1. Material properties of pavement materials

| Property | HL content | Pavement layer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Wearing | Levelling | Base | Subbase | Subgrade |
| Modulus E (MPa) | 2.5% | 0.1898T2 – 43.03T + 2623 | 0.5248T2 – 63.3T + 2527 | 0.3359T2 – 41.12T + 1803 | 170 | 65 |
| Poisson ratio u |  | 0.35 | 0.35 | 0.35 | 0.4 | 0.4 |
| Density ρ (g/cm3) |  | 2.32 | 2.30 | 2.297 | 1.76 | 1.29 |
| Thermal Conductivity k (W/m/K) |  | 0.8989 | 0.7757 | 0.8763 | 1.3 | 0.28 |
| Thermal Capacity cp (J/kg/K) |  | 1333.48 | 1155.49 | 1303.09 | 837 | 800 |
| Thermal deformation coefficient α |  | CTE CTC | CTE CTC | CTE CTC | 3.32e-6 | 3.4e-5 |

- FE Analysis of the Pavement using Hydrated Lime Modified Mixes

  - A pavement Finite Element model

A single lane carriageway pavement structure of five layers as illustrated in Fig. 4 is designed for a numerical finite element analysis. Fig. 4 shows only a half of the symmetric pavement structure. A wheel load applied to represent the traffic. The geometric information has been listed in the Table 2.

Fig. 4. FE geometric model

Table 2. FE geometric data

|  | Wearing | Levelling | Base | Subbase | Subgrade | Wheel |
| --- | --- | --- | --- | --- | --- | --- |
| Depth (mm) | 50 | 70 | 90 | 300 | 2500 | 0 |
| Width (mm) | 1800 | 1800 | 1800 | 1800 | 1800 | 250 |

  - Climatic conditions

The thermal interaction between pavement and local atmospheric environment has been a topic of interest in pavement engineering and urban environment research. The thermal exchange mechanism with climatic atmosphere at the pavement surface consists of three physics, i.e., the solar irradiation adsorption, pavement emission and interfacial air convection [Wu et al. 2018, Qin 2016, Chen et al 2019]. It decides the vertical thermal flux at the pavement surface, which is quantified by the Eq. (8):

qT=1-aI-εσTs4-Ta4-hc(Ts-Ta)					(8),

where qT (W/m2) stands for the vertical thermal flux at the pavement surface, a is the albedo coefficient of the pavement material, I (W/m2) is the solar incident irradiation, ε is pavement material emissivity, σ is the Stefan-Boltzmann constant, 5.6710-8 W/m2/K4, hc (W/m2/K) is the heat convection coefficient.

For the climatic thermal effect analysis, a scenario has been assumed at a location in Greater Manchester in the northwest England. A database of the UK Global Horizontal Irradiance (GHI) has been referred to determine the thermal boundary condition for the FE modelling of the pavement structure [Palmer et al. 2017]. It records the annual hourly incident irradiation around the UK. To characterise these data for the FE modelling parametric input, at first, an average daily variation by hours in each month was calculated. Fig. 5 gives out the result of a set of the data at a location of the identification - GHI_tmy_MIDAS X_394936,Y_392020 2008_2017 [Palmer and Betts. 2018]. A Gaussian distribution (Eq. 9) was employed to represent the monthly daily average hourly GHI in a period of one year. Thereafter, the three representative Gaussian parameters, a, b and c were characterized using a sum sine function as presented in the Fig. 6.

GHI=a×exp⁡(- t - bc2)							(9),

where t is the time in hours, a, b and c are three parameter constants.

Fig. 5. Annual monthly average daily Global Horizontal Irradiance at the location in Greater Manchester UK

Fig. 6. Characterized Gaussian representative parameters

To determine the qT by the Eq. (8), we need to know the atmospheric air temperature Ta, for which the data from UK weather forecast record was adopted for this paper. Fig. 7(a) shows the annual monthly temperature variation in the region of Manchester and a representative curve fitting for the average of the Low and High. Fig. 7(b) shows the variation magnitude, i.e. the difference between the Average and the Low or High, and a representative curve fitting to the data of the difference. On the data and the representatives in Fig. 7, the hourly atmospheric air temperature condition is characterised using the Eq. (10) below and illustrated in the Fig. 8.

Ta= Tmean+DT×sin⁡(2t/24)					(10),

where Tmean is the average curve in the Fig. 7(a), and DT=High-Low2 is the variation magnitude against the average and represented by the curve in the Fig. 7(b), t is the time in hours. The x in both Fig. 7(a)&(b) is the time in months, i.e., x = t/30/24.

| (a) | (b) |
| --- | --- |

Fig. 7. UK Manchester area annual temperature variation

Fig. 8. Characterized hourly air temperature variation in one year time

Many heat convection models have been proposed for the hc in the Eq. (8) [Chen et al. 2019], which directly relates to the local atmospheric weather state, such as temperature, humidity and wind [Bentz 2000, Dempsey et al. 1985]. In this paper, a constant value, 13.5 W/m2/°C, was assumed, a data according to [Qin & Hiller 2013].

To solve the energy equation, Eq. (6), in FE modelling, we also need to define the thermal boundary condition. For the symmetric line, a thermal insulation is applied. For the boundary temperature Tb, a geological survey on the shallow ground seasonal temperature in the UK [Busby 2015] is assumed to be the surrounding ground condition for this work. Fig. 9 shows the data and characterization fitting curves for the temperature variation at five depths underground, i.e., 0, -0.5, -1, -2 and -3m. An initial temperature condition in the structure is assumed to be 8.7 °C, taking the mean value at the depth -0.5m.

Fig. 9. Year-round UK shallow ground temperature variation [Busby 2015]

  - Traffic condition

A wheel pressure applied on the surface represents the traffic loading. An equivalent single wheel load of 70kN [CivilWeb Spreadsheets] is set for the modelling work, which gives a wheel load pressure of 0.772 MPa on an estimated contact area of 9.07310-2 m2 [Al-Tameemi, et al. 2019]. The boundary condition to solve the motion equation, Eq. (2), assumes that there is no horizontal displacement for the vertical edges of all layers, and no vertical displacement at the bottom of the subgrade

- Modelling Results

Fig. 10 shows the calculated pavement surface temperature variation. It can be seen that it presents a similar patten as the atmospheric air temperature variation shown in Fig. 8, but the maximum value is about 2.5C lower than that of the atmospheric air temperature. Fig. 11 shows the vertical deformation of the pavement surface at for different hours in a year. It can be seen that the HL modified pavement shows less deformation.

Fig. 10. Pavement surface temperature variation in year round

Fig. 11. Comparison of the vertical deformation of pavement surface

Fig. 12 shows the temperature profile in the depth of the pavement structure at four times. It can be seen that the pavement structure, in a year-round time, has the biggest temperature variation at the surface. Fig. 13 compares the maximum thermal strain and the two principle mechanical strains in the pavement structure. It can be seen that the thermal effect has a noticeable influence on the 1st principle mechanical strain. Meanwhile, the 2nd principal strain is very small.

Fig. 12. Temperature profile along depth

c

Fig. 13. the maximum thermal strain and mechanical strain in pavement structure

Fig. 14 shows the calculated daily average fatigue life consumption, 1/Nf, at the position on the pavement surface, the A, B, C & D in Fig. 4. The Nf is estimated using Eq. (1), where the εt_200th is simply taken the value of the sum of the thermal stain, α∆T, and the 1st principal stain, εmax=εxx+εyy2+εxx-εyy22+εxy2. The total fatigue life loss in one year time due to thermal effect or temperature variation can be approximated by integrating the curves in the Fig. 14. It is not surprise that the highest fatigue exposure is at the location under the coupled thermal and mechanical impact and in the season of summer. The integrated value for the position B is 0.0088, i.e. the thermal effect on the whole material fatigue life loss is about 0.88% in one year period of time.

Fig. 14. an estimation for the fatigue life consumption at the pavement surface

Conclusion

This paper presents a study on the thermal effect on the asphalt concrete using hydrated lime for partial mineral filler. The experiment test has confirmed that a 2.5% hydrated lime replacement for the conventional limestone filler generates an optimum fatigue life result for the mix used for wearing course application. A follow-on numerical modelling for a scenario assumed in the Manchester area in northwest of England provides a quantitative evaluation on the local climate condition effect on the life of the constructed pavement. The case study modelling demonstrates a considerable climatic fluence on pavement fatigue. Continuing research is undergoing to compare the fatigue life of pavements using and without use of hydrated lime modified concrete.

References

Al-Tameemi A. F., Wang Y., & Albayati A., 2016, Experimental Study of the Performance Related Properties of Asphalt Concrete Modified with Hydrated Lime. Journal of Materials in Civil Engineering, 28(5).

Al-Tameemi, A. F. et al. ,2019, Moisture Susceptibility and Fatigue Performance of Hydrated Lime–Modified Asphalt Concrete: Experiment and Design Application Case Study, Journal of Materials in Civil Engineering. American Society of Civil Engineers, 31(4), p. 4019019.

Albayati A. and Mahir A., 2015, Temperature Prediction Model for Asphalt Concrete Pavement’, in LJMU 14th Annual International Conference on Asphalt, P. E. and I. (ed.). Liverpool.

Bourona S., Hammouma F., Ruatb H., Métaisc P., Lesueurc D., 2021, Case study - Improving the durability of asphalt mixtures with hydrated lime: Field results from highway A84, Case Studies in Construction Materials 14, e00551

Busby J., 2015, UK shallow ground temperatures for ground coupled heat exchangers, Quarterly Journal of Engineering Geology and Hydrogeology, 48, 248-260

Chen J., Wang H., Xie P., 2019, Pavement temperature prediction: Theoretical models and critical affecting factors, Applied Thermal Engineering 158: 113755

CivilWeb Spreadsheets - Engineering Calculations & Spreadsheets - Equivalent Single Wheel Load, https://civilweb-spreadsheets.com/road-pavement-design/equivalent-single-wheel-load/

Huang Y., 2004, Pavement Analysis and Design, Pearson Prentice Hall, Upper Saddle River, N.J.

Islam M.R. and Tarefder R.A., 2015, Coefficients of Thermal Contraction and Expansion of Asphalt Concrete in the Laboratory, Journal of Material in Civil Engineering, 27(11): 04015020-1

Kai L. and Fang W. 2011, Computer modeling mechanical analysis for asphalt overlay under coupling action of temperature and loads’, Procedia Engineering, 15: 5338–5342.

Lesueur D., Petit J., and Ritter H.J., 2013, The mechanisms of hydrated lime modification of asphalt mixtures: A state-of-the-art review, Road Mater. Pavement Des. 14 (1): 1–16.

Little D.N., Allen D.H, Bhasin A., 2018, Modeling and Design of Flexible Pavements and Materials, Springer International Publishing AG, Switzerland

Palmer D. and Betts T., 2018, Uk-wide Typical Meteorological Year Global Horizontal Irradiance Data for Photovoltaic Systems, https://doi.org/10.17028/rd.lboro.6945311.v1

Palmer D., Cole I., Betts T., Gottschalg R., 2017, Interpolating and estimating horizontal diffuse solar irradiation to provide UK-wide coverage: selection of the best performing models, Energies, Special Issue - Solar Photovoltaics Trilemma: Efficiency, Stability and Cost Reduction, 10, 181.

Qin Y., Pavement surface maximum temperature increases linearly with solar absorption and reciprocal thermal inertial, International Journal of Heat and Mass Transfer 97 (2016) 391–399

Souza, F.V and Castro, L.S., 2012, Effect of temperature on the mechanical response of thermo-viscoelastic asphalt pavements, Construction and Building Materials, 30: 574–582.

Xu Q. and Solaimanian M, 2010, Modeling temperature distribution and thermal property of asphalt concrete for laboratory testing applications, Construction and building materials, 24(4): 487–497.

SCRB/R9, 2003, General specification for roads and bridges, Ministry of Housing and Construction, Republic of Iraq.

Wu H., Sun B.i, Li Z., Yu J., 2018, Characterizing thermal behaviors of various pavement materials and their thermal impacts on ambient environment, Journal of Cleaner Production 172: 1358e1367

CONTENTS OF PREVIOUS ISSUES

May 2000 – Volume 1 Issue 1

TECHNICAL PAPERS

A unified approach to reporting and evaluating asphalt characteristics

Nunn,  C.R.T., United Kingdom

Optimum design of rigid road pavements using Austroads

Hadi, M.N.S., Australia

Backcalculation of flexible pavement layers moduli and bonding conditions between wearing and base courses

Al-Nageim, H.K. & Al-Hakim, B., United Kingdom

Highway performance modelling in Hungary

Gaspar, L., Hungary

Edge drainage details for porous asphalt

Santhalingam, V. & Nicholls, J.C., United Kingdom

TECHNICAL NOTES

Waste materials in pavement engineering

Chan, W.W.J. & Lawrence, C.M., Hong Kong

Introduction to the current types of asphalt surfacings

Nicholls, J.C., United Kingdom

November 2000 - Volume 1 Issue 2

TECHNICAL PAPERS

Upgrading low quality aggregates for use in flexible pavement

Al-Nageim, H.K., United Kingdom

Subgrade failure criterion for the design of flexible pavements based on higher wheel loads

Hadi, M.N.S., Australia

Preparation of samples for wheel tracking test

Gibney, A., Webster, T. & O’Callaghan, R., Ireland

Techniques for the rehabilitation of concrete pavements in tropical, developing countries

Parry, J.D. & Freer-Hewish, R.J., United Kingdom, Beaty, A.N.S., Canada

May 2001 - Volume 2 Issue 1

TECHNICAL NOTES

A brief introduction to bitumen viscosity / rheology and why it is important

Reid J.M., United Kingdom

TECHNICAL PAPERS

Dynamic shear rheometers: measurement, accuracy, precision and prediction

Carswell, J., United Kingdom

Laboratory study of the resistance of bituminous overlays to reflective cracking

Gibney, A. Lohan, G. & Moore, V., Ireland

A new test for assessing the durability of asphalt mixtures for severe winter conditions

Hobeda, P., Sweden

Life cycle cost analysis of pavements

Hadi, M.N.S., Australia

Effect of lower grade aggregates on the performance of concrete surfacing

Collins, R.J., United Kingdom.

November 2001 - Volume 2 Issue 2

TECHNICAL PAPERS

The performance and behaviour of stone element pavements

Fordyce, D. & Khweir, K., United Kingdom

Developing specifications for unsealed road pavements

Muhammed, M.N.S., Australia

Optimisation of bituminous pavement thickness in mechanistic pavement design

Narasimham, K.V., Misra, R & Das, A., India

Cellulose oil palm fibre as a binder stabiliser in stone mastic asphalt

Muniandy, R., Vasudevan, J & Omar, H. India

October 2002 - Volume 3 Issue 1

TECHNICAL PAPERS

Evaluation of effective pavement layer moduli using genetic algorithms

Reddy, M.A., Reddy, K.S. & Pandey, B.B., India

Sustainable pavements

Walsh, I.D., United Kingdom

The design and laying of a road trial in Thessaloniki for road markings

Oikonomou, N., Greece, Nicholls, J.C. & Woodside, A.R., United Kingdom, Delivopoulos, I., Greece

A model for the prediction of the elastic stiffness of bituminous mixtures under pulse loading

Santagata, E. & Bassani, M., Italy

January 2003 - Volume 3 Issue 2

TECHNICAL PAPERS

Design of high modulus composite pavements

James, D.J., MacGregor, I.D. & Hammond, J.E., United Kingdom

Microsurfacing of concrete roads in Romania: case histories

Oikonomou, N., Papakatiskas, C. & Dimpoulos, K., Greece

Prediction for pumping in concrete pavements on cement-treated base

Kuo, C-M. & Huang, Y.T. Taiwan

European co-operation in pavement research using accelerated loading test facilities – an introduction

Nunn, M.E., United Kingdom, Hildebrand, G., Denmark

May 2003 - Volume 4, Issue 1

TECHNICAL NOTES

Overcoming barriers to recycling

Barritt, J., United Kingdom

TECHNICAL PAPERS

The use of neural networks for backcalculation of layer moduli

Gossain, V. & Reddy, K.S., India

Cold Recycling (SMART project)

Carswell, I., United Kingdom

A low-cost falling weight deflectometer for pavement evaluation in India

Reddy, M.A., Kumar, R.S. & Reddy, K.S., India

LANDREC – a remediation technology for re-using contaminated soil

Phillips, P., United Kingdom, Hildebrand, G., Denmark

December 2003 - Volume 4, Issue 2

TECHNICAL PAPERS

Using Drucker-Prager Model for the Analysis of Recycled Road Pavements

Hadi, M.N.S. and Symons, M.G., Australia

Models for Estimation of Sub grade Moduli from DCP Tests

Kumar, R.S., Reddy, K.S., Mazumdar, M. and Pandey, B.B., India

Back calculation of Pavement Layer Moduli from FWD Data Using Genetic Algorithm

Manish K., Maheshwari, N. and Das, A., India

Finite Element Implementation and Verification of an Elastoplastic Model for Unsaturated Soils

Nesnas, K., United Kingdom

May 2004 - Volume 5, Issue 1

TECHNICAL NOTES

Cold paving in the UK: a Case Study

Day, D. & Staples, S., United Kingdom

TECHNICAL PAPERS

Pavement Condition before and after Rehabilitation

Gáspár, L., Hungary

Prediction of Flexible Pavement Layer Moduli in India

Kumar, R.S. & Reddy, K.S. India

Developments in Sustainable Asphalt Technology

Thom, N.H., United Kingdom

Exploitation and Use of Quarry Fines: A Preliminary Report

Woods, S., Mitchell, C.J., Harrison, D.J. & Manning, D.A.C.

December 2004 - Volume 5, Issue 2

TECHNICAL NOTES

DTI/ WRAP Research: Wastes and Resources Action Programme

TECHNICAL PAPERS

Ground Penetrating Radar for Monitioring of English Roads

Forest, R. Pynn, J. Alani, A. & Ferne, B. United Kingdom

A Revised Mechanistic Method for Design of Indian Highways

Kumar, R.S. & Reddy, K.S. India

Design of Flexible Pavements Based on Finite Element Analysis

Bodhinayake, B.C. & Hadi MNS. Australia

Reservoir Pavements: A solution of Sustainable Urban Drainage

Nunn, M.E. United Kingdom

May 2005, Volume 6, Issue 1

TECHNICAL PAPERS

Practical Aspects Surrounding a Full-Scale Implementation of a Performance- Based Specification for New Road Construction in the United Kingdom

I. Widyatmoko, R.C. Elliot, J. McCulloch and J. Norman, UK

Bottom Ash Embankments and Their Long-Term Performance

L. Gaspar, Hungary

A continuing Study of the Skid Resistance Performance of BOS Slag Used as a Road Surface Course Aggregate

P.G. Roe, UK

A Review of Recent Research Into Sustainability Pavement Materials and Techniques

I. Walsh, UK

December 2005, Volume 6, Issue 2

TECHNICAL PAPERS

The Use Of Natural Flax Fibres In Stone Mastic Asphalt

G. Airey (UK), J. O. Vidana Vencomo (Mexico) & J. Harwood (UK)

How Surplus Quarry Dust Is Being Put To Good Use In New Applications In South Wales

M. Lamb & M. Reid (UK)

A Proposed Quantitative Model For Highway Repair And Maintenance

A. Alani. A. Teo & J. Goulding (UK)

The Feasibility Of Recycling Thin Surfacing Systems

J. C. Nicholls (UK)

May 2006, Volume 7, Issue 1

TECHNICAL PAPERS

The Influence of Ageing on Dry Process Crumb Rubber Modified Asphalt Mixtures

M. Rahman (UK), G. Airey (UK) and A. Collop (UK)

Structural Evaluation of a Concrete Filled Cell Pavement

U.C. Sahoo, K.S. Reddy, B.B. Pandey (India)

Laboratory Measurement of Bond Between Asphalt Layers

M. Sutanto (UK), A. Collop (UK), G. Airey (UK), R. Elliott (UK) and Y. Choi (Australia)

Reuse of Thermally Purified Tar- Containing Asphalt

A.R.G. Vanderwall (The Netherlands)

December 2006, Volume 7, Issue 2

TECHNICAL PAPERS

Laboratory Evaluation Of Cement Kiln Dust (CKD) As Filler In Asphalt

Cliff Nicholls (TRL Ltd, UK) John Lay (CEMEX UK Materials Limited) and Murray Reid (TRL Ltd, UK)

Effect Of Shelf Life Time On The Performance Of Foamed Bitument Bound Mixtures

Kadhim Khweir (Pavement Technology Limited, Heriot-Watt University, UK)

A Review Of Geotechnical Specfications For Sustainable Infrastructure

P.R Fleming, M.W Frost, J.P Lambert (Loughborough University)

A Life Cycle Approach To The Selection Of Construction Materials

Y Huang, R Bird (Newcastle University), B Allen (Aggregate Industries, UK)

May 2007, Volume 8, Issue 1

TECHNICAL PAPERS

Laboratory investigation of recycled binder performance

Laurent Porot and Dr. Sophie Nigen-Chaidron, Shell Bitumen, France

Evaluation of roller compacted concrete pavement with bottom ash by ultrasound measurements – an experimental study

Glicério Trichês , Roberto C. A. Pinto and Alexandre José Silva

Universidade Federal de Santa Catarina, Brazil

Procedures for identifying hazards in component materials for asphalt

Cliff Nicholls – TRL Ltd,  UK

Virginie Mouillet – Régional des Ponts et Chaussées d’Aix-en-Provence, France

François Deygout – Shell Bitumen, France

Burgard Koenders – Shell Bitumen, France

Piouslin Samuel – TRL Ltd, UK

Technical feasibility of the use of eva discarded by the footwear industry  in hot-mix asphalt

Jesner Sereni Ildefons and, Jose Leomar Fernandes Jr., University of Sao Paulo, Brazil

Laboratory Investigation of Recycled Binder Performance

Laurent Porot and Dr. Sophie Nigen-Chaidron, Shell Bitumen, France

December 2007, Volume 8, Issue 2

TECHNICAL PAPERS

Laboratory Evaluation of TBM Super a Crumb Rubber Modified Bitumen and Waste Plastics Modified Bitumen Using Superpave Methology For Indian Conditions B.V. Kiran Kumar, Dr. H.S. Jagadeesh, Dr. R. Sathyamurthy and Dr. Sunil Bose

A Review of the Development of Pervious Pavements and Sustainable Urban Drainage (SUDS) in the UK C. J. Pratt, Emeritus Professor of Stormwater Management, Coventry University, Coventry

Innovation in the Production and Commerical use of Energy Extracted from Asphalt Pavements C. Sullivan, Managing Director, Material Edge Ltd Arian De Bondt & Rob Jansen, Ooms International Holding bv Henk Verweijmeren, Invisible Heating Systems Ltd

Numerical Modelling of Asphalt Mixture Wheel Tracking Experiments Dr Alistair Hunter, Prinicipal Engineer, Scott Wilson, Nottingham Dr Gordon Airey, Nottingham Centre for Pavement Engineering, University of Nottingham Ouahid Harireche, Nottingham Centre for Pavement Engineering, University of Nottingham

May 2008, Volume 9, Issue 1

TECHNICAL PAPERS

A Sustainable Hot Binder for Surface Dressing – With Improved Performance

Bert Jan Lommerts, Martin R. Verweij and Huig van Duijn Latexfalt B.V., the Netherlands Evaluation of Roller Compactor Cum Rut Analyzer an Alternative Compactor For Bituminous Mix Design B.V. Kiran Kumar, Dr. H.S. Jagadeesh, Dr. R. Sathyamurthy and Dr. Sunil Bose, Dayananda Sagar College of Engineering, Bangalore-50 078, Karnataka, India Using Coal Fly Ash in Road Construction Lindon K. A. Sear, UK Quality Ash Association, UK Reach Ahead – The Implications of the New Chemicals Regulation within Europe for the Bitumen & Asphalt Industry M. Southern, Eurobitume, Belgium

December 2008, Volume 9, Issue 2

TECHNICAL PAPERS

Pollutant Potential Of Soil Stabilized With Bottom Ash To Be Used In Pavement Structures Glicerio Triches, Edney Rodrigues de Farias, Marciano Maccarini, Federal University of Santa Catarina, Brazil Sustainability Through Durability Of Asphalt Cliff Nichols, TRL Ltd., UK The Influence Of Pavement Layers Behaviour On Thickness Design Using Kenlayer Program Amir Kavussi, Ali Babazadeh, Tarbiat Modares University, Iran Use Of Bitumen Emulsion And Worn Mobile Tyres In The Production Of Concrete Paving Blocks Nikolaos Oikonomou, Sofia Mavridou, Aristotle University of Thessaloniki (A.U.Th), Greece

May 2009, Volume 10, Issue 1

TECHNICAL PAPERS

| Durable Pavement Maintenance Using Reinforced Asphalt, 20 Years of Experience J. L. Walter, Group Asphalt Solutions Manager, Tensar International Limited, Blackburn, UK N. Reck, Application Technology Manager, Tensar International Corporation, Atlanta, USA E. Luce, Produktmanager Asphalteinlagen, Tensar International GmbH, Germany Measuring and Evaluating the Characteristics of Asphalt at Low Temperatures C. Karcher, Universität Karlsruhe (TH), Institute of Highway and Railroad Engineering, Germany K. Mollenhauer, Braunschweig Pavement & Engineering Centre, Technische Universität Braunschweig, Germany Simple Indices For Rapid Evaluation of Workability and Rutting Resistance of Indonesian Wearing Course Mixtures I. Haryanto -University of Gadjah Mada Jalan Yacaranda, Indonesia O. Takahashi, Nagaoka University of Technology, Japan Effects of Grading of Fine Aggregate on the Strength and Permeability of Concrete Used In Pavement Construction K. Thamer, A.S.K. Kwan & B.I.G. Barr – Cardiff School Of Engineering, Cardiff University, UK |
| --- |
| December 2009, Volume 10, Issue 2 TECHNICAL PAPERS |  |  |
| Sustainable Maintenance of a UK Trunk Road Using Crack and Seat and Ex Situ Cold Mix Recycling – A Case Study P. Langdale, I. Carswell Transport Research Laboratory, Wokingham, UK T. McHugh, Cemex UK Construction Limited, Worsley, UK M. Ashfield, Cemex UK Construction Limited, Worsley, UK |  |  |
| Development of Best Practice for Recycling Surface Course Materials Back Into Thin Surfacing I. Carswell, Transport Research Laboratory, Wokingham, UK J. C. Nicholls, Transport Research Laboratory, Wokingham, UK I. Widyatmoko, Scott Wilson Limited, Nottingham, UK J. Harris, Lafarge Aggregates Limited, Lutterworth, UK R. Taylor, Shell Bitumen Limited, Manchester, UK D. Lee, Highways Agency, Bedford, UK |  |  |
| Permeable Pavements - A Case Study Milton Keynes Park and Ride Sukalpa Biswas, Pavement Engineer, Mouchel, Birmingham, UK |  |  |
| Mechanical Characterization and Influence of Water On WMA By Adding Synthetic Zeolite M. Bocci, Full Professor - Università Politecnica delle Marche, Ancona, Italy A. Grilli, Ph.D. - Università Politecnica delle Marche, Ancona, Italy V. Pannunzio, Ph.D. Student – Università Politecnica delle Marche, Ancona, Italy P.P. Riviera, Ph.D. – Politecnico di Torino, Abruzzi, Torino |  |  |
| May 2010, Volume 11, Issue 1 |  |  |  |
| TECHNICAL PAPERS |  |  |  |
| MECHANICAL AND FRACTURE MECHANICAL PROPERTIES OF ASPHALT GEOSYNTHETIC INTERLAYER SYSTEMS Thomas Hasslacher, Eng. & Rainer Lugmayr, C. Eng., TenCate Geosynthetics, A-4021 Linz, Schachermayerstr. 18, Austria |  |  |  |
| IMPROVING THE DURABILITY OF LONGUITUDINAL JOINTS IN ASPHALT PAVEMENTS Ignacio Artamendi, Bob Allen, Chris Allpress, Daniel Morgan, Aggregate Industries, Research & Development Department, Ashbourne, Derbyshire ,UK. Chris Sullivan, Material Edge, 9 Wheat Moss, Chelford, Macclesfield, Cheshire ,UK |  |  |  |
| THE USE OF X-RAY CT TO CHARACTERIZE RUTTING IN UK ASPHALT MIXTURES Andrew Dawson, Rawid Khan, Andrew C Collop and Yang Lintao, Nottingham Transportation Engineering Centre (NTEC), University Park, Nottingham,UK |  |  |  |
| TERMINAL CONDITION FOR THIN SURFACED LOW VOLUME ROADS U. C. Sahoo, Department of Civil Engineering, Birla Institute of Technology, Mesra, Ranchi, India K. S. Reddy, Department of Civil Engineering, Indian Institute of Technology, Kharagpur, India |  |  |  |

December 2010, Volume 11, Issue 2

TECHNICAL PAPERS

Future Developments in Asphalt Technology From A German Viewpoint Michael P. Wistuba, ISBS Institut für Straßenwesen, Braunschweig Pavement Engineering Centre, Germany

Strength Properties of Roller Compacted Concrete Containing Rice Husk Ash Joe Arnaldo Villena Del Carpio, Glicério Trichês and Luiz Roberto Prudêncio Jr., Department of Civil Engineering, Universidade Federal de Santa Catarina, Brazil

Swedish Pavement Performance Specification Safwat F. Said, Swedish National Road & Transport Research Institute (VTI)

Subgrade Strength Characterization For Low Volume Roads In India U. C. Sahoo and K. S. Reddy, Department of Civil Engineering, Indian Institute of Technology, Kharagpur, India

May 2011, Volume 12, Issue 1

TECHNICAL PAPERS

DYNAMIC FIELD ASSESSMENT OF SHORT TERM BEARING CAPACITY FOR COLD RECYCLED LAYERS Alessandro Marradi, Dynatest Denmark A/S, Naverland 32 – DK 2600 Glostrup, Denmark

FROM VEHICLE / SURFACE INTERACTION TO QUIET SURFACE DRESSINGS David Woodward, Phillip Millar and Shaun Friel, School of the Built Environment, Shore Road, Newtownabbey, Co. Antrim, Northern Ireland

CEMENTITIOUS ACTIVITY OF CALCINED RED MUD Daniel Véras Ribeiro, Marcio Raymundo Morelli, Department of Materials Engineering / Federal University of São Carlos, Rodovia Washington Luis, Km 235. 13565-905. São Carlos/SP, Brazil

FLUSHING SLAG BOUND MIXTURE IN THE UK John Kennedy, Pavement Engineering Consultant, UK

December 2011, Volume 12, Issue 2

| TECHNICAL PAPERS |  |
| --- | --- |
| #### EVALUATION OF AIRPORT PAVEMENTS USING A COMBINED INDEX Rafiqul A. Tarefder, Raju Bisht and Mesbah U. Ahmed, Department of Civil Engineering, University of New Mexico, USA. |  |
| THE ROLE OF DURABILITY OF SURFACING IN HIGHWAY ASSET MANAGEMENT Prof. Ian. D. Walsh, Road Consultants Ltd , UK |  |
| COMPARATIVE STUDY OF EMPIRICAL AND MECHANISTIC-EMPIRICAL PAVEMENT DESIGN METHODOLOGY USING KENLAYER SOFTWARE M. Waseem Mirza, Imran Hafeez and Mumtaz A. Kamal, Department of Transportation Engineering and Management,University of Engineering and Technology, Lahore, Pakistan |  |
| LABORATORY PREDICTION OF THIN SURFACING EARLY LIFE ASPHALT SURFACING PROPERTIES Shaun Friel, David Woodward and Alan Woodside, University of Ulster, Jordanstown, Northern Ireland |  |

May 2012, Volume 13, Issue 1

| TECHNICAL PAPERS |  |
| --- | --- |
| MONITORING THE PERFORMANCES IN REAL WORKING CONDITIONS OF C & D WASTE MATERIALS IN ROAD CONSTRUCTION USING EMBEDDED INSTRUMENTS Mauro Coni, James Rombi, Valentina Zedda, Silvia Portas, Salvatore Pistis, Faculty of Engineering, University of Cagliari, Italy |  |
| GB5®: AN ALTERNATIVE TO EME2 FOR LONG-LASTING AND COST-EFFECTIVE ASPHALT CONCRETES BY USE OF AGGREGATE PACKING CONCEPTS AND SBS MODIFIED BINDERS François Olard, EIFFAGE Travaux Publics, R&D Project Manager, 8 rue du Dauphiné BP 357, F-69960 Corbas Cedex, France |  |

December 2012, Volume 13, Issue 2

| TECHNICAL PAPERS |  |
| --- | --- |
| EVALUATION OF MOISTURE SUSCEPTIBILITY IN HOT MIX ASPHALT PAVEMENT USING A MODEL MOBILE LOAD SIMULATOR Mansour Solaimanian, Senior Research Associate, Penn State University,USA. Chieh-Tang Chang, Engineer, CECI Engineering Consultants, Inc., Taiwan Ghassan Chehab, Assistant Professor, American University of Beirut, Beirut, Lebanon. EFFECTS OF AIRVOIDS ON LOW TEMPERATURE FRACTURE STRENGTH OF ASPHALT CONCRETE Adnan Qadir, Professor, NED University, Karachi Murat Guler, Associate Professor, Middle East Technical University, Dumlupinar Blvd. EQUIVALENT LINEAR ELASTIC MODULUS OF GRANULAR LAYER FOR LOW VOLUME ROADS Umesh Chandra Sahoo, Assistant Professor, Indian Institute of Technology Bhubaneswar, India K. Sudhakar Reddy, Professor, Indian Institute of Technology, India. AUTOMATIC DETECTION AND ANALYSIS OF SURFACE DISTRESSES USING A MULTI FUNCTIONAL VEHICLE Alessandro Marradi, Senior Engineer – Dynatest Denmark A/S, Naverland, Denmark Kars Drenth, Head of Pavement Engineering – Dynatest Denmark A/S, Naverland, Denmark Martyn Stonecliffe-Jones, Senior Engineer – Dynatest UK Ltd.. |  |

May 2013, Volume 14, Issue 1

| TECHNICAL PAPERS |  |
| --- | --- |
| AN EVALUATION METHOD FOR THE PERFORMANCE OF STRESS ABSORBING MEMBRANE INTERLAYERS (SAMIs) AGAINST REFLECTIVE CRACKING UNDER THERMAL LOADING O.M. Ogundipe,Civil Engineering Department, Ekiti State University Ado-Ekiti, Nigeria N.H.Thom, NTEC, University of Nottingham, UK A.C.Collop,Faculty of Technology, De-Montfort University, UK. DEVELOPING THE USE OF RECYCLED AND SECONDARY AGGREGATES IN QATAR Dr K E Hassan, TRL QSTP-B, Doha, Qatar Dr M S Al-Kuwari, Qatar Standards, Ministry of Environment, Doha, Qatar Dr J M Reid, A Berhane and C Collis, TRL, Wokingham, Berkshire,UK. ASSESSMENT OF WATER AND TEMPERATURE SUSCEPTIBILITIES OF COLD RECYCLED BITUMINOUS EMULSION MIXTURES USING THE NAT IN THE ITSM MODE OF TESTING O. L. Oke,Lecturer - Ekiti State University - Civil Engineering Department, Faculty of Engineering, Ekiti State University, P.M.B. 5363, Ado–Ekiti, Ekiti State, Nigeria T. Parry, Associate Professor – University of Nottingham - Nottingham Transportation Engineering Centre, University of Nottingham, Nottingham, UK N. H. Thom,Lecturer – University of Nottingham - Nottingham Transportation Engineering Centre, University of Nottingham, Nottingham, UK. A NEW PRESERVATION TECHNOLOGY FOR POROUS ASPHALT - A MULTI-MODAL, BIOPOLYMER STABILIZED, REJUVENATING ASPHALT EMULSION Bert Jan Lommerts, Latexfalt bv, P.O. Box 6, 2396 ZG Koudekerk aan den Rijn, the Netherlands Jan Struik, Latexfalt bv, P.O. Box 6, 2396 ZG Koudekerk aan den Rijn, the Netherlands Irina Cotiugă, Latexfalt bv, P.O. Box 6, 2396 ZG Koudekerk aan den Rijn, the Netherlands Gerbert van Bochove, Breijn bv, Heijmans R&D Laboratories, Rosmalen, the Netherlands. December 2013, Volume 14, Issue 2 TECHNICAL PAPERS. EVALUATION OF TEST METHODS AND SELECTION OF AGGREGATE GRADING FOR TYPE III APPLICATION OF MICRO-SURFACING Masoud Robati, M.Eng, PhD Student, École de technologie supérieure (ÉTS), Montreal, Canada Alan Carter,Professor (ÉTS) Daniel Perraton, Professor (ÉTS) COMPARISON STUDY BETWEEN LIME APPLICATION METHODS ON HMA ASPHALT CONCRETE IN IRAQ Amjad H. K. Albayati, Assist prof. Transportation. Engineering, University of Baghdad Ahmed M. M. Alani, M.Sc Candidate Transportation Engineering, University of Baghdad MECHANICAL PERFORMANCE OF BITUMINOUS CONCRETE INCORPORATING STEEL SLAG WITH NATURAL AGGREGATE Satish Pandey, Scientist, CSIR-Central Road Research Institute, Mathura Road, New Delhi 110025, India Dr. P.K.Jain, Chief Scientist, CSIR-Central Road Research Institute, Mathura Road,New Delhi 110025, India EVALUATION OF WARM-MIX ASPHALT MIXTURES USING LEADCAP ADDITIVE Kim, Yongjoo, Ph. D, Senior Researcher, Dept. of Highway Division, Korea Institute of Construction Technology, 1190, Simindae-Ro, Ilsanseo-Gu, Goyang-Si, Gyeonggi-Do, Republic of Korea Lee, Jaejun, Baek, Cheolmin, Kwon, Sooahn, Suh, Youngchan, Professor, Department of Transportation Engineering, Hanyang University, Republic of Korea. Son, Jongcheol, Director, Road Planning & Construction Division, Ministry of Land, Transport and Maritime Affairs, Gyeonggi-Do, Republic of Korea May 2014, Volume 15, Issue 1 TECHNICAL PAPERS. COMPARATIVE STRUCTURAL ANALYSIS OF FLEXIBLE PAVEMENTS USING FINITE ELEMENT METHOD Ankit Gupta, Assistant Professor, Department of Civil Engineering, NIT Hamirpur, India Abhinav Kumar, Former Graduate Student, Department of Civil Engineering, NIT Hamirpur, India COHESION, FRACTURE AND BOND - UNDERSTANDING THE DATA FROM THE VIALIT COHESION PENDULUM TEST AND OTHER FRACTURE TESTS FROM AN ANALYSIS OF RHEOLOGICAL PROPERTIES Geoffrey M. Rowe Abatech Inc., PO Box 356, Blooming Glen, PA 18911, U.S.A PERFORMANCE CHARACTERISTICS OF FIBER MODIFIED ASPHALT CONCRETE MIXES Manoj Shukla, Senior Scientist, Central Road Research Institute (CRRI), New Delhi, India Dr. Devesh Tiwari, Principal Scientist, Central Road Research Institute (CRRI), New Delhi, India K. Sitaramanjaneyulu, Senior Principal Scientist, Central Road Research Institute (CRRI), New Delhi, India EXPERIMENTAL ANALYSIS OF WATERPROOFING POLYMERIC PAVEMENTS FOR CONCRETE BRIDGE DECKS Marco Pasetto, Professor, University of Padua, Department of Civil, Environmental and Architectural Engineering - Padova, Italy Giovanni Giacomello, Ph.D. student, University of Padua, Department of Civil, Environmental and Architectural Engineering - Padova, Italy. December 2014, Volume 15, Issue 2 TECHNICAL PAPERS ANALYSIS OF RUNWAY DEFLECTOMETER CAMPAIGN FOR IMPLEMENTATION ON AIRPORT PAVEMENT MANAGEMENT SYSTEM Franco Pigozzi, Silvia Portas, Francesca Maltinti and Mauro Coni, Department of Civil, Environmental and Architectural Engineering, University of Cagliari, Italy CORRELATIONS BETWEEN AGGREGATE PARTICLE SHAPE PARAMETERS AND SIZE BASED ON IMAGES CAPTURED USING A 3-D LASER SCANNING DEVICE Martin B Mgangira and Julius Komba, CSIR Built Environment, P.O Box 395, Pretoria, 0001, South Africa SITE TRIALS OF RECYCLED AND SECONDARY AGGREGATES IN CONCRETE AND CONCRETE BLOCKS IN QATAR Dr. K. E. Hassan, TRL, QSTP-B, Doha, Qatar Dr. M. S. Al-Kuwari, Qatar Standards, Ministry of Environment, Doha, Qatar Dr. J. M. Reid, Senior Technical Manager, Infrastructure, TRL, Crowthorne House, Nine Mile Ride, Wokingham, Berkshire RG40 3GA, United Kingdom INVESTIGATING FILLER CHARACTERISTICS IN UPGRADING COLD BITUMINOUS EMULSION MIXTURES Dr. Shakir Al-Busaltan, Lecturer, Department of Civil Engineering, College of Engineering, University of Kerbala, Kerbala, Iraq May 2015, Volume 16, Issue 1 TECHNICAL PAPERS. WARM MIX ASPHALT FOR AUSTRALIAN AIRPORTS Greg White Technical Manager Airports, Fulton Hogan 2404 Logan Road, Eight Mile Plains, Brisbane, Australia INVESTIGATION AND DESIGN OF DURABLE PAVEMENT STRUCTURE REHABILILATION Márton Karoliny Senior Advisor, EULAB Ltd., 1 Székesdűlő Str. Dunakeszi, Hungary, 2120 László Gáspár Research Professor, KTI Non-profit Ltd., 3-5 Thán Károly Str. Budapest, Hungary, 1119 DEVELOPMENT OF A PAVEMENT RUTTING MODEL USING SHAKEDOWN THEORY Mostapha Boulbibane, Faculty of Engineering, Sports and Sciences, University of Bolton, BL3 5AB, UK Ian F. Collins, Department of Engineering Science, University of Auckland, New Zealand THE STRESS RELAXATION OF MODIFIED BITUMENS Pavel Coufalík, Ondřej Dašek, Jiří Kachtík and Jan Kudrna, Brno University of Technology, Faculty of Civil Engineering, Czech Republic Svatopluk Stoklásek, RNDr., Independent consultant, Czech Republic LABORATORY TRIALS OF COLD RECYCLED FOAMED BITUMEN ASPHALT: RAF WADDINGTON Helena Isabel Lacalle Jiménez and Jessica Tuck, URS Infrastructure & Environmental UK Limited, 12 Regan Way, Chetwynd Business Park, Chilwell, Nottingham NG9 6RZ, UK December 2015, Volume 16, Issue 2 TECHNICAL PAPERS INVESTIGATION OF HOT IN-PLACE RECYCLING EFFECTS ON HOT MIX ASPHALT PAVEMENT Dr Nadeem Anwer Qureshi & Dr Bilal Khurshid, Military College of Engineering (MCE), National University of Science and Technology (NUST), Risalpur Campus, Pakistan ESTIMATION OF JOINT AND INTERFACE PARAMETERS FOR THE FINITE ELEMENT ANALYSIS OF JOINTED CONCRETE PAVEMENT USING STRUCTURAL EVALUATION RESULTS Swati Roy Maitra, K. S. Reddy & L. S. Ramachandra, Civil Engineering Department, Indian Institute of Technology, Kharagpur, Pin 721 302, West Bengal, India EFFECT OF AGGREGATE GRADATION ON THE STIFFNESS OF ASPHALT MIXTURES Hasan Al-Mosawe, Nick Thom & Gordon Airey, Nottingham Centre for Pavement Engineering, School of Civil Engineering, University of Nottingham, University Park, Nottingham NG7 2RD, UK Amjad Al-Bayati, Department of Civil Engineering, University of Baghdad, Baghdad, Iraq VARIABILITY IN PAVEMENT DESIGN Paola Dalla Valle, Senior Engineer, Arup, The Arup Campus, Blythe Valley Park, Solihull, B90 8AE, UK Dr Nick Thom, Lecturer, Department of Civil Engineering, Nottingham Centre for Pavement Engineering, University of Nottingham, NG7 2RD, UK May 2016, Volume 17, Issue 1 TECHNICAL PAPERS. PERFORMANCE VARIATION DUE TO AIR VOID DISTRIBUTION IN OPEN-GRADED FRICTION COURSE PAVEMENTS Dr Nadeem Anwer Qureshi & Dr Bilal Khurshid, Military College of Engineering (MCE), National University of Science and Technology (NUST), Risalpur Campus, Pakistan AIRPORT ASPHALT SURFACE INTERFACE SHEAR RESISTANCE: FACTORS AFFECTING AND ADVANCED CHARACTERISATION Greg White & Tom Gabrawy, Fulton Hogan Australia, 2404 Logan Road, Eight Mile Plains, Queensland, Australia. ANALYSIS OF THE LINEAR VISCOELASTIC BEHAVIOUR OF BITUMEN WITH HIGH QUANTITIES OF FILLER Professor Marco Pasetto & Giovanni Giacomello, University of Padua, Department of Civil, Environmental and Architectural Engineering, Via F. Marzolo, 9, 35131, Padova, Italy LIFETIME ENGINEERING PRINCIPLES AND DURABLE ROADS Laszlo Gaspar, KTI (Institute for Transport Sciences), Budapest, Hungary December 2016, Volume 17, Issue 2 TECHNICAL PAPERS INVESTIGATIONS OF PREMATURE FAILURE OF AN ASPHALT OVERLAY Cherif Amer-Yahia, Julia Miller, Todd Majidzadeh, Chhote Saraf & Kamran Majidzadeh – USA THE IMPACT OF RAINFALL DURING ASPHALT PAVING OPERATIONS Greg White – Australia REQUIRED MECHANICAL PROPERTIES OF A CLEAR BINDER FOR COLOURED ASPHALT CONCRETE Kees Plug & Arian De Bondt – Netherlands A NEW AND INNOVATIVE STEEL-BASED ANTI-REFLECTIVE CRACKING INTERLAYER F. Vervaecke & H. Cornelus - Belgium & P. Straubinger – Germany May 2017, Volume 18, Issue 1 TECHNICAL PAPERS STUDY ON EFFECT OF VICOSITY OF FOAMING CHARACTERISTICS AND STABILIZED MIX PROPERTIES Siksha Swaroopa Kar, Dr P K Jain, Dr A K Swammy & Dr. Devesh Tiwari - India CLIMATE RESILIENCE OF FLEXIBLE PAVEMENT HIGHWAYS: ASSESSMENT OF CURRENT PRACTICE Yaning Qiao, Dermot B Casey, Kranthi K Kuna, Kieran Kelly, Bingling Mei, & Iain D MacGregor – United kingdom EFFECT OF LABORATORY COMPACTION METHOD ON THE MECHANICAL PROPERTIES OF BITUMINOUS MATERIALS C. Allpress, I. Artamendi, B. Allen & P. Phillips – United Kingdom INCREASED DAMAGE TO UPHILL FLEXIBLE PAVEMENTS FROM 11.22+2.22 FULL-TRAILER TRUCKS Sabah Said Razouki, Raid R. Al-Muhanna , & Zahraa H. M. Al-hashimi - Iraq December 2017, Volume 18, Issue 2 TECHNICAL PAPERS ACCELERATED DURABILITY TESTING USING THE IMMERSION AGEING TEST FOR THIN ASPHALT SURFACINGS Chibuzor Ojum, Iswandaru Widyatmoko, Martin Heslop, & Arash Khojinian WARM CHEMICAL ADDITIVES TO IMPROVE WATER RESISTANCE OF ASPHALT MIXTURES CONTAINING STEEL SLAGS: A MULTI-SCALE APPROACH Marco Pasetto, Emiliano Pasquini, Giovanni Giacommello, & Andrea Baliello “GREEN” PAVEMENT OVERLAYS. PART I: FLEXURAL PERFORMANCE OF COMPOSITE BEAMS ON ELASTIC FOUNDATION J.N. Karadelis, Y. Lin, & Y. Xu “GREEN” PAVEMENT OVERLAYS. PART II. SHEAR PERFORMANCE OF COMPOSITE BEAMS ON ELASTIC FOUNDATION AND THEIR NUMERICAL REPRESENTATION J.N. Karadelis, Y. Xu, & Y. Lin May 2018, Volume 19, Issue 1 TECHNICAL PAPERS CHARACTERIZING THE CRACK RESISTANCE PROPERTIES OF HALF-WARM BITUMINOUS EMULSION MIXTURES INCORPORATED CRUSHED WASTE GLASS Mustafa Amoori Kadhim, Shakir Al-Busaltan, & Raid R. Almuhanna, Iraq CALIBRATION OF HDM-4 PAVEMENT PERFORMANCE MODELS FOR INDIAN NATIONAL HIGHWAYS Sanjay Deori, Rajan Choudhary, Devesh Tiwari, & S. Gangopadhyay, India A NOVEL LABORATORY TEST METHOD TO MEASURE DYNAMIC WATER PRESSURE UNDERNEATH A CRACKED CONCRETE PAVEMENT Fauzia Saeed , Mujib Rahman, & Denis Chamberlain, UK EVALUATING THE DEFORMATION RESISTANCE OF HALF-WARM BITUMEN EMULSION MIXTURE Muna Al- AL-Kafaji, Shakir Al-Busaltan, & Hussein A. Ewadh, Iraq December 2018, Volume 19, Issue 2 TECHNICAL PAPERS ASSESSING THE IMPACT OF POLYMER ADDITIVES ON DEFORMATION AND CRACK HEALING OF ASPHALT CONCRETE SUBJECTED TO REPEATED COMPRESSIVE STRESS Saad Issa Sarsam & Sara Ali Jasim, Iraq ENHANCING STABILITY OF CLAYEY SUBGRADE MATERIALS WITH CEMENT KILN DUST STABILIZATION Magdy A. Abd El-Aziz & Mosatfa A. Abo-Hashema, Egypt SKIDDING RESISTANCE: MEASUREMENT AND USE OF DATA Mark Stephenson, UK INVESTIGATION OF THE EFFECTS OF ADDITIVES ON MOISTURE SUSCEPTIBILITY OF ASPHALT MIXES CONTAINING SULFUR-POLYMER Amir Kavussi, Mozhgan Jahantighi & Javad Bakhtiari, Iran May 2019, Volume 20, Issue 1 TECHNICAL PAPERS MONITORING AND SELF-HEALING OF SURFACE-INITIATED CRACKS IN GAP-GRADED ASPHALT PAVEMENTS Miguel A. Franesqui, Juan Gallego IMPROVED METHODS FOR THE IMPUTATION OF MISSING DATA IN PAVEMENT MANAGEMENT SYSTEMS Pedro Marcelino, Maria de Lurdes Antune, Eduardo Fortunato, Marta Castilho Gomes PERFORMANCE EVALUATION OF FOAMED BITUMEN BOUND MIXTURES MADE WITH RECYCLED AND ARTIFICIAL AGGREGATES AND FIBRES Marco Pasetto , Giovanni Giacomello, Andrea Baliello , Emiliano Pasquini COMPARATIVE ANALYSIS OF EFFECT OF HIGHER AXLE LOADING AND TYRE PRESSURES ON PAVEMENT PERFORMANCE OF FLEXIBLE PAVEMENT USING IITPAVE & KENPAVE Abhishek Sharma, Tanuj Chopra March 2020, Volume 21, Issue 1 TECHNICAL PAPERS REFLECTIVE CRACKING IN INVERTED PAVEMENTS: FINDINGS FROM SIMULATIVE LABORATORY TESTS Imtiaz Ahmed, Nick Thom, Syed Bilal Ahmed Zaidi, Andrew Dawson MODERNISATION OF THE DESIGN MANUAL FOR ROADS AND BRIDGES (DMRB) VOLUME 7, PAVEMENT DESIGN & MAINTENANCE Dave Gershkoff, Martin Greene, Matt Wayman, Peter Langdale, Arash Khojinian USE OF WASTE TYRE PYROLYTIC PRODUCTS FOR ASPHALT BINDER MODIFICATION Abhinay Kuma and Rajan Choudhary CHARACTERIZATION OF SEASONAL VARIATIONS ON AIRPORT PAVEMENT LAYER RESPONSES USING IN-SITU MEASUREMENTS Richard Y. Ji, P.E., Ph.D DEVELOPMENT OF OMAN PERFORMANCE GRADE PAVING MAP FOR SUPERPAVE ASPHALT MIX DESIGN Mostafa A. Abo-Hashema, Ragab M. Mousa, Samiha Abdulrahim Al-Zedjali, Qais Abdullah Al Balushi, Mohamed Metwally, and Majid Hamood Al-Rashdi BITUMINOUS BINDERS EXTENDED WITH A RENEWABLE PLANT-BASED OIL: TOWARDS A CARBON NEUTRAL BITUMEN Xiaohu Lu, Carl Robertus, Jenny-Ann Östlund RAVELLING RESISTANCE OF UK THIN ASPHALT SURFACE COURSE SYSTEMS UNDER A LARGE-SCALE ACCELERATED TEST METHOD Giacomo D’Angelo, Chibuzor Ojum, Iswandaru Widyatmoko, Matthew Wayman, Arash Khojinian DEVELOPING THE CASE FOR EXTENDING SERVICE LIFE OF ASPHALT SURFACINGS USING HYDRATED LIME Dr Rebecca Hooper, Dr Helen K Bailey, Richard Givens, Mike Haynes, and Darren Scutt PERFORMANCE EVALUATION OF COMPOSITE ASPHALT BINDER Nawazish Ali, I. Hafeez, M.A. Baluch SIMULATING THE INFLUENCE OF SURFACE FREE ENERGY ON MOISTURE DAMAGE OF RECYCLED ASPHALT CONCRETE Saad Issa Sarsam, Rana Khalid Hamdan EVALUATING OF AGING BEHAVIOR OF THIN ASPHALT OVERLAY MODIFIED WITH SUSTAINABLE MATERIALS Nadia Abduljabbar, Shakir Al-Busaltan, Anmar Dulaimi, Ola Al Jawad FIELD PERFORMANCE OF RECYCLED AND ALTERNATIVE AGGREGATE IN CONCRETE Dr K E Hassan, IRD QSTP-LLC, Doha, Qatar March 2021, Volume 21, Issue 2 TECHNICAL PAPERS |  |

REFLECTIVE CRACKING IN INVERTED PAVEMENTS: FINDINGS FROM SIMULATIVE LABORATORY TESTS

Imtiaz Ahmed, Nick Thom, Syed Bilal Ahmed Zaidi, Andrew Dawson

MODERNISATION OF THE DESIGN MANUAL FOR ROADS AND BRIDGES (DMRB) VOLUME 7, PAVEMENT DESIGN & MAINTENANCE

Dave Gershkoff, Martin Greene, Matt Wayman, Peter Langdal, Arash Khojinian

USE OF WASTE TYRE PYROLYTIC PRODUCTS FOR ASPHALT BINDER MODIFICATION

Abhinay Kumar and Rajan Choudhary

CHARACTERIZATION OF SEASONAL VARIATIONS ON AIRPORT PAVEMENT LAYER RESPONSES USING IN-SITU MEASUREMENTS

Richard Y. Ji, P.E., Ph.D

DEVELOPMENT OF OMAN PERFORMANCE GRADE PAVING MAP FOR SUPERPAVE ASPHALT MIX DESIGN

Mostafa A. Abo-Hashema, Ragab M. Mousa, Samiha Abdulrahim Al-Zedjali, Qais Abdullah Al Balushi, Mohamed Metwally, and Majid Hamood Al-Rashdi

BITUMINOUS BINDERS EXTENDED WITH A RENEWABLE PLANT-BASED OIL: TOWARDS A CARBON NEUTRAL BITUMEN

Xiaohu Lu, Carl Robertus, Jenny-Ann Östlund

RAVELLING RESISTANCE OF UK THIN ASPHALT SURFACE COURSE SYSTEMS UNDER A LARGE-SCALE ACCELERATED TEST METHOD

Giacomo D’Angelo, Chibuzor Ojum, Iswandaru Widyatmoko, Matthew Wayman, Arash Khojinia

DEVELOPING THE CASE FOR EXTENDING SERVICE LIFE OF ASPHALT SURFACINGS USING HYDRATED LIME

Dr Rebecca Hooper, Dr Helen K Bailey, Richard Givens, Mike Haynes, and Darren Scutt

PERFORMANCE EVALUATION OF COMPOSITE ASPHALT BINDER

Nawazish Ali, I. Hafeez, M.A. Baluch

SIMULATING THE INFLUENCE OF SURFACE FREE ENERGY ON MOISTURE DAMAGE OF RECYCLED ASPHALT CONCRETE

Saad Issa Sarsam, Rana Khalid Hamdan

EVALUATING OF AGING BEHAVIOR OF THIN ASPHALT OVERLAY MODIFIED WITH SUSTAINABLE MATERIALS

Nadia Abduljabbar, Shakir Al-Busaltan, Anmar Dulaimi, Ola Al Jawad

FIELD PERFORMANCE OF RECYCLED AND ALTERNATIVE AGGREGATE IN CONCRETE

Dr K E Hassan, Dr M S Al-Kuwari, Dr J M Reid

March 2021, Volume 22

TECHNICAL PAPERS

ASSESSING THE VARIABILITY OF DEFORMATION UNDER REPEATED TENSILE AND SHEAR STRESSES FOR ASPHALT CONCRETE

Saad Issa Sarsam

EFFECTIVENESS OF TACK COATS AND BOND COATS IN ASPHALT PAVEMENTS

Dr Robert N Hunter

IMPACTS OF LOW-DENSITY POLYETHYLENE (PLASTIC SHOPPING BAGS) ON STRUCTURAL PERFORMANCE AND PERMEABILITY OF HMA MIXTURES

Chayanon Boonyuid, Dr. Shohel Amin MCIHT FHEA

STUDY ON PRODUCTION TEMPERATURES OF ASPHALT BINDERS MODIFIED WITH WASTE TYRE PYROLYTIC CHAR

Abhinay Kumar, Rajan Choudhary and Ankush Kumar

ASSET MANAGEMENT IN ACTION

WORLD PREMIERE FOR COLAS - PORTSMOUTH PFI & ISO 55001 CERTIFICATION

Shine Salur, MSc, MPhil, CEng MICE MCIHT

Information for Prospective Authors

Manuscripts in the format detailed below should be sent as a word document via email to:

Professor Al Nageim

Liverpool John Moores University

E-mails: h.k.alnageim@ljmu.ac.uk

Dr. Anmar Dulaimi

Liverpool John Moores University

E-mails: a.f.dulaimi@ljmu.ac.uk

Assist. Prof. Dr. Shakir Al-Busaltan

University of Kerbala, Iraq

E-mail: s.f.al-busaltan@uokerbala.edu.Iq

Copyright

Authors submitting manuscripts for publication warrant that the work is not an infringement of any existing copyright and will indemnify the publisher against any breach of such warranty. Papers and contributions published in the Journal become the legal copyright of the publisher unless otherwise agreed in writing. The publisher will not refuse any reasonable request by the author for permission to reproduce any contributions made.

Manuscript form and refereeing procedure

- All papers should be written in English and submitted in Microsoft Word format.

- All papers will be independently refereed by at least two members of the Editorial Review Board or other acknowledged expert in the field.

- The font should be Times New Roman, font size 12, with single spacing used throughout the document, save for the title.

- Page margins: top and bottom 1 inch; left and right 1.25 inches with full justification. The opening page should contain the title (font: Times New Roman, font size: 15; format: bold, centred and in capital letters).

- Double space down for author’s name, brief affiliation and address, (use upper and lower case). The address should include street, building etc., town, country, post/zip code, telephone, facsimile and email details.

- Leave three blank lines and then type Abstract (centred, in bold, capital letters – the style for all main headings). Space down two – begin abstract of approximately 250 words that should concisely describe content.

- Sub-headings are bold, initial cap only, left justified, immediately above the paragraph.

- Papers should not exceed 8,000 words (approximately 14 pages).

- Use standard A4 sized paper (the US 8 ½ x 11-inch paper size is also acceptable).

- Use the SI system for all scientific and laboratory data. Temperature should be in degrees Celsius.

- If you include a photograph, figures, or extract which has been published, written consent must be obtained from the original publisher and/or person who retains the copyright for reprints. All illustrations and tables should be placed in the text near to the point where they are referred to or at the end of the paper. Tables and drawings should be produced by a computer or drawn on white paper with black waterproof drawing ink.

- Tables should be labelled at the top left corner. Example: Table 1. Title initial cap only (bold).

- Figures should be captioned beneath, and centred: Figure 1. Effect of plate…. Also initial cap and bold.

- References within the text should be cited with parenthesis without numbering: (Al-Nageim, 1992) or (Morley and Couch, 1987). For more than two authors, use the name of the first author followed by et al., (Al-Nageim et al, 2000). Two or more references cited together should be separated by semicolons (Al-Nageim, 1998; Morley 2000).

- References should be listed in alphabetical order at the end of the document under references (centred and bold). The authors surname should be listed first, followed by a comma and initial(s). All authors of a given reference should be listed followed by: (i) year of publication in parenthesis e.g. (2001); (ii) title of “article” in quotation marks, journals and books italicised [books should include name(s) of editor(s), publisher, and place of publication. For journals include volume, year/month of issue]. Page numbers should comprise the initial page and the number of the final page e.g. pp 45-48 or p.12.

- Papers or other contributions and the statements made or opinions expressed therein are published on the understanding that the author is solely responsible for the opinion expressed therein and that its publication does not imply that such statements and/or opinions reflect the views or opinions of the Editors, Editorial Review Board or Publisher.

Subscription Form

The International Journal of Pavement Engineering and Asphalt Technology is published twice a year. The subscription rates are:

£120.00 annual subscription (electronic copy/s)

£150.00 annual subscription (hard copy/s inclusive of postage) £60.00 single issue (electronic copy)

£80.00 single issue (hard copy inclusive of postage) £30.00 single paper (electronic copy)

Please use the online payment by the:

https://buyonline.ljmu.ac.uk/product-catalogue/ljmu/faculty-shop/faculty-of-engineering-technology/the-international-journal-of-pavement-engineering-and-asphalt-technology

Alternatively, please complete and send the subscription form to the address below.

Total Cost (£)

Name:

Company Name:

Address:

Telephone:

Facsimile:

E-mail:

By Cheque Only Payable in £ from a UK Bank account

Please return this form with your payment to the address specified below (cheques should be drawn on a British bank and be made payable to Liverpool John Moores University):

Professor Hassan Al Nageim

Department of Civil Engineering

Liverpool John Moores University

Room 2.01 Peter Jost Building

Byrom Street

Liverpool

L3 3AF

UK

If you have any subscription enquiries please contact us at h.k.alnageim@ljmu.ac.uk

Instructions to make online payment for subscription

- Go to the university website: www.ljmu.ac.uk

- Choose online payments (click on the right at the bottom).

- Choose LJMU online shop (click on the left).

- Click on the option for all other products.

- Choose product catalogue.

- Choose faculty shop (click on the left).

- Click on technology & environment.

- Select International Journal of Pavement Engineering and Asphalt Technology, PEAT.

- Choose your desierd products (annual subscription, single issue, single paper in hard or electronic format).

- Choose the quantity of products.

- Select add to basket and the select proceed to basket.

- Choose proceed to checkout and finish payment following the instructions.

N.B.

- You will get the information and confirmation of your payment through your e-mail address.

- Payments are accepted by credit or debit card - Visa, MasterCard.

- No American Express please.

Call for Papers

The International Journal of Pavement Engineering & Asphalt Technology has been established as a leading refereed forum. It reports on innovative research; design breakthroughs; construction practices; evaluation techniques; industrial applications; specification & analysis of asphalt binders & mixtures.  Authors are invited to submit papers for inclusion in future editions. Theoretical papers should deal with innovation and, wherever possible, the implementation of new materials or techniques. Papers describing work in progress should discuss the long-term practical implications of the research undertaken. Contributions are especially welcome from designers and contractors dealing with new materials or construction / testing techniques (including destructive and non-destructive testing). Papers discussing IT, construction management, environmental maintenance and pavement material performance will also be considered.

The International Journal of Pavement Engineering & Asphalt Technology (ISSN: 1464-8164) is published twice yearly. All rights including translation reserved. The Editors and members of the Journal’s Editorial Review Board are from a range of organisations including academic institutions, oil and chemical producers, government institutions, consulting companies and equipment developers with a wide range of expertise.

Papers or other contributions and the statements made or opinions expressed therein are published on the understanding that the author of the contribution is solely responsible for the opinion expressed in it and its publication does not imply that such statements and / or opinions are, or reflect the views or opinions of, the Editors, the Editorial Review Board or the Publisher. The views expressed in the Journal do not represent any of the organisations for which the Editors or members of the Editorial Review Board are associated.

ISSN 1464-8164           March 2022

Volume 23                                                 ay 2010

The International Journal of Pavement Engineering & Asphalt Technology